PYTHON IMPLEMENTATION GUIDE

Step-by-step code for UAV multimodal sensing of white rust in mustard: from orthomosaics to severity and spray maps

Pre-processing · co-registration · vegetation masking and soil removal · endmember extraction and unmixing · plot-wise features · disease-specific feature selection · single-sensor models · multi-sensor fusion · fusion models · SHAP · maps

Dataset: 4 sensors (hyperspectral, multispectral, RGB, thermal) × 4 flight dates × 48 plots (12 healthy control, 36 inoculated)

Animesh Panda

Ph.D. Scholar, Division of Agricultural Physics, ICAR–IARI, New Delhi

Companion to the methodology document · October 2026

# How to use this guide

This guide contains complete, runnable Python code for every step of the analysis, in the order in which the steps are run. Each chapter explains what the step does, which inputs it needs, which parameters to adjust, what it writes, and how to check that the result is sensible, followed by the full code of that step.

The same code is saved as separate .py files in the folder rustwatch_pipeline/ of the project repository. Copy the code from the `.py` files rather than from this document, because word processors can change quotation marks and indentation. The whole pipeline was run end to end on a synthetic dataset with exactly your structure before this guide was written (Chapter 12).

Your data, as assumed by the code 4 sensors: hyperspectral (HS), multispectral (MS, MicaSense RedEdge-MX), RGB, thermal (TIR). 4 flight dates per sensor, i.e. 16 orthomosaics. 48 plots on every image: 12 uninoculated control plots (healthy, plots 1–12) and 36 inoculated plots (R1 = plots 37–48, R2 = 25–36, R3 = 13–24) with different severities. Analysis unit = one plot on one date: 48 × 4 = 192 observations. Cross-validation is always grouped by plot, so the 4 dates of a plot are never split between training and testing.

Steps added to your list (needed for valid results) Field layout check and plot polygons (Step 1): the polygons drive every later step; the code also checks the treatment layout. Ground-truth labels (Step 1): Disease Index → six severity classes, with a check for natural infection in control plots. Radiometric and thermal conversion, empirical-line correction (Step 2): features are only comparable across dates when images are in reflectance and °C. Hyperspectral bad-band removal and Savitzky–Golay smoothing (Step 2). Co-registration between sensors (Step 2): sub-pixel shift estimation so that masks and plots line up on every sensor. Shadow removal (Step 3) and a low-cover fallback (Step 5) so that severely diseased plots are not lost from the analysis. Quality control of features (Step 6): missing values, near-zero variance, outliers. Redundancy removal (VIF, SPA) before SFFS (Step 7). Nested, plot-grouped cross-validation (Step 7): without it, reported accuracy is optimistically biased. Statistical comparison of models (McNemar), fusion gain, SHAP explanation (Step 8). Severity and spray-zone maps and regression of the continuous Disease Index (Step 8).

Check your field layout The layout slide shows replication R1 (plots 37–48) with D4V1 and D2V2 twice and D2V1 and D4V2 missing. R2, R3 and the control block each contain all 12 treatments once. The code prints this warning in Step 1. Correct the TREATMENTS table in s01_layout.py if the slide has a typing error; if the field really was sown this way, note it in the thesis.

# Contents

Right-click the table of contents in Word and choose “Update Field” to show page numbers.

# 1. Set-up

## 1.1 Install Python and the packages

Use Python 3.10 or newer. Create a separate environment so that package versions do not clash with other work:

python -m venv rw_env

# Windows:  rw_env\Scripts\activate        Linux / macOS:  source rw_env/bin/activate

pip install numpy scipy pandas scikit-learn scikit-image rasterio geopandas shapely \

statsmodels mlxtend shap matplotlib

The code was tested with Python 3.11, numpy 2.4, scipy 1.17, pandas 3.0, scikit-learn 1.9, scikit-image 0.26, rasterio 1.4, geopandas 1.2, statsmodels 0.15, mlxtend 0.25 and shap 0.51. On Windows, installing rasterio and geopandas is easiest with conda: conda install -c conda-forge rasterio geopandas.

## 1.2 Folder structure

All inputs and outputs live under rustwatch_pipeline/project/. Each step reads the previous step’s folder and writes its own:

| Folder / file | Content | Written by |
| --- | --- | --- |
| 01_orthomosaics/<sensor>/<sensor>_<date>.tif | Your 16 orthomosaics (input) | You |
| plots.gpkg, field_layout.csv | Plot polygons and treatment table | Step 1 |
| ground_truth.csv | plot_id, date, DI (input) | You |
| weather.csv | date, air_temp_C at flight time (input, for CTD/CTR) | You |
| hs_wavelengths.txt | One wavelength (nm) per HS band, if not in an ENVI header (input) | You |
| calibration_panels.csv | Optional panel values for the empirical line (input) | You |
| 02_preprocessed/ | Reflectance / °C rasters, co-registered; coregistration_log.csv | Step 2 |
| 03_masks/ | Vegetation masks per sensor and date | Step 3 |
| 04_unmixing/ | Endmember spectra (CSV, PNG) and abundance rasters | Step 4 |
| 05_features/ | features_MS.csv, features_HS.csv, features_RGB.csv, features_TIR.csv | Step 5 |
| 06_dsf/ | Phase 1 tables per sensor, JM scalogram, QC report | Step 6 |
| 07_models/ | Outer-CV predictions, performance summary, fusion gain, OFCs, SHAP | Steps 7–8 |
| 08_maps/ | Severity and spray maps (PNG) and predictions (GeoPackage) | Step 8 |

## 1.3 Input file formats

ground_truth.csv                 weather.csv              calibration_panels.csv (optional)

plot_id,date,DI                  date,air_temp_C          sensor,date,band,dn_dark,dn_bright,refl_dark,refl_bright

1,2026-01-20,0.0                 2026-01-20,20.0          MS,2026-01-20,0,1840,21500,0.05,0.48

37,2026-01-20,4.2                2026-01-25,22.5          MS,2026-01-20,1,1920,22900,0.05,0.49

...                              ...                      ...

Dates must be written exactly as in config.DATES (YYYY-MM-DD). The Disease Index is computed from the 0–6 leaf rating scale as in the ORW (DI = Σ(x·f) / (n·Σf) × 100).

## 1.4 Running the pipeline

cd rustwatch_pipeline

python s01_layout.py            # or: python run_all.py        (all steps)

python run_all.py 2 3 4 5       # selected steps

RW_FAST=1 python run_all.py 7   # quick test of the modelling (fewer trees, 1 repeat)

Steps 1–6 take minutes. Step 7 (nested cross-validation of 8 sensor configurations × 3 classifiers) takes the longest; run it first with RW_FAST=1 to check that everything works, then without it for the thesis results.

# 2. Step 0: Configuration (config.py)

Every path, date and parameter is set in this one file. Before running anything with your data, change: the four DATES; the CRS if your orthomosaics are not in UTM zone 43N; FIELD_ORIGIN_E, FIELD_ORIGIN_N and FIELD_ROTATION_DEG (the RTK coordinates of the north-west corner of plot 48 and the field orientation) if you build the plot polygons from the layout; and the MicaSense band order if your export differs.

| Parameter | Default | Meaning |
| --- | --- | --- |
| PLOT_LENGTH_M, PLOT_WIDTH_M | 5.0, 3.5 | Plot size from the layout slide |
| CHANNEL_M | 2.0 | Irrigation channels after plot rows 1 and 3 |
| PLOT_INSET_M | 0.4 | Inward buffer so plot edges and bunds are excluded |
| DI_CLASS_BINS | 0, 1–5, 6–10, 11–25, 26–50, >50 | DI (%) → severity classes 0–5 (ORW) |
| HS_BAD_RANGES | <400, 1340–1460, 1790–1960, >2450 nm | Noisy and water-absorption bands removed |
| N_ENDMEMBERS | 4 | Soil, healthy canopy, diseased canopy, shadow |
| FDR_Q, MIN_DATES_SIGNIFICANT | 0.05, 2 | Feature must be FDR-significant on ≥ 2 of 4 dates |
| JM_MIN_HS_BAND, JM_MIN_DERIVED | 1.94, 1.0 | Separability thresholds (ORW; methodology document) |
| VIF_MAX | 10 | Redundancy threshold |
| SFFS_MAX_FEATURES | 10 | Largest subset SFFS explores |
| OUTER_FOLDS, INNER_FOLDS, N_REPEATS | 4, 4, 3 | Nested CV: 4 outer folds of 12 plots each, repeated 3 times |

File: rustwatch_pipeline/config.py

"""Step 0 - Project configuration. Edit this file only; every other module reads from it."""

from pathlib import Path

# ---------------------------------------------------------------- folders

ROOT = Path(__file__).resolve().parent / "project"

RAW = ROOT / "01_orthomosaics"        # input: one orthomosaic per sensor per date

PRE = ROOT / "02_preprocessed"        # calibrated, co-registered rasters

MASK = ROOT / "03_masks"              # vegetation masks

UNMIX = ROOT / "04_unmixing"          # endmembers and abundance maps

FEAT = ROOT / "05_features"           # plot-wise feature tables

DSF = ROOT / "06_dsf"                 # Phase 1 outputs

MODELS = ROOT / "07_models"           # Phase 2 and 3 outputs

MAPS = ROOT / "08_maps"               # severity and spray maps

PLOTS_FILE = ROOT / "plots.gpkg"            # plot polygons (Step 1)

LAYOUT_CSV = ROOT / "field_layout.csv"      # plot_id, row, col, treatment, rep

GROUND_TRUTH_CSV = ROOT / "ground_truth.csv"  # plot_id, date, DI

WEATHER_CSV = ROOT / "weather.csv"            # date, air_temp_C at flight time

# ---------------------------------------------------------------- campaign

SENSORS = ["MS", "HS", "RGB", "TIR"]

DATES = ["2026-01-20", "2026-01-25", "2026-01-30", "2026-02-04"]  # replace with your 4 flight dates

CRS = "EPSG:32643"   # WGS 84 / UTM zone 43N (Delhi)

# File naming convention: RAW/<sensor>/<sensor>_<date>.tif  (HS may also be ENVI .hdr/.dat)

def raw_path(sensor, date):

return RAW / sensor / f"{sensor}_{date}.tif"

# ---------------------------------------------------------------- sensors

# MicaSense RedEdge-MX band order and centre wavelengths (nm)

MS_BANDS = {"B": 475, "G": 560, "R": 668, "RE": 717, "NIR": 840}

RGB_BANDS = ["R", "G", "B"]

# Hyperspectral: wavelengths are read from the ENVI header if present,

# otherwise from this file (one wavelength in nm per line).

HS_WAVELENGTH_FILE = ROOT / "hs_wavelengths.txt"

HS_BAD_RANGES = [(0, 400), (1340, 1460), (1790, 1960), (2450, 3000)]  # nm removed

SG_WINDOW, SG_POLY = 11, 2               # Savitzky-Golay smoothing

REFERENCE_SENSOR = "MS"   # all sensors are resampled onto this sensor's grid for masking

# ---------------------------------------------------------------- field layout

PLOT_LENGTH_M = 5.0       # along the field's 24 m side

PLOT_WIDTH_M = 3.5        # along the field's 48 m side

N_COLS, N_ROWS = 12, 4

FIELD_WIDTH_M, FIELD_LENGTH_M = 48.0, 24.0

CHANNEL_M = 2.0           # irrigation channels after plot rows 1 and 3

PLOT_INSET_M = 0.4        # inward buffer to avoid edge effects

# Field corner of the top-left plot (row 1, column 1) and field orientation.

# Measure with RTK GNSS; rotation is degrees anticlockwise from east.

FIELD_ORIGIN_E = 712000.0

FIELD_ORIGIN_N = 3170024.0

FIELD_ROTATION_DEG = 0.0

# ---------------------------------------------------------------- labels

# DI (%) -> severity class, as in the ORW (0 healthy ... 5 very high)

DI_CLASS_BINS = [-0.01, 0.0, 5, 10, 25, 50, 100]

CLASS_NAMES = ["Healthy", "Very low", "Low", "Moderate", "High", "Very high"]

# ---------------------------------------------------------------- masking

NDVI_MIN = None           # None = automatic Otsu threshold

SHADOW_NIR_PERCENTILE = 5 # darkest NIR pixels treated as shadow

RGB_EXG_MIN = None        # None = Otsu on ExG

# ---------------------------------------------------------------- unmixing

N_ENDMEMBERS = 4          # soil, healthy canopy, diseased canopy, shadow

UNMIX_SAMPLE = 20000      # pixels sampled for endmember extraction

# ---------------------------------------------------------------- feature selection

FDR_Q = 0.05

MIN_DATES_SIGNIFICANT = 2     # of the 4 dates

JM_MIN_DERIVED = 1.0

JM_MIN_HS_BAND = 1.94

RELIEF_K = 10

BOOTSTRAP_N = 50

STABILITY_MIN = 0.6

VIF_MAX = 10.0

SFFS_MAX_FEATURES = 10

OUTER_FOLDS, INNER_FOLDS = 4, 4

N_REPEATS = 3

RANDOM_STATE = 2026

# 3. Step 1: Field layout, plot polygons and ground truth (s01_layout.py)

What it does. Stores the plot numbers and treatments exactly as on the layout slide, assigns each column group to its block (R1, R2, R3, CONTROL), checks that every block contains each of the 12 treatments once, and builds one polygon per plot from the plot size, the 0.55 m gaps between columns, and the two 2 m irrigation channels. It also reads the ground truth, converts DI to the six severity classes, and warns if any control plot has DI > 0.

If you already digitised the plots in QGIS or ArcGIS, skip the polygon builder: save your layer as project/plots.gpkg with an integer plot_id column. Digitised polygons are usually more accurate than a computed grid. Either way, overlay plots.gpkg on an orthomosaic in QGIS and confirm that every polygon sits on its plot before continuing.

Output. field_layout.csv, plots.gpkg, and the class counts per date printed on screen. Check that each date has enough observations per class; with 48 plots some classes may hold only a few plot-dates.

File: rustwatch_pipeline/s01_layout.py

"""Step 1 - Field layout, plot polygons and ground-truth labels."""

import numpy as np

import pandas as pd

import geopandas as gpd

from shapely.geometry import Polygon

from shapely import affinity

import config as C

# Plot numbers and treatments exactly as in the field-layout slide (row 1 = top/north row).

PLOT_IDS = [

[48, 41, 40, 33, 32, 25, 24, 17, 16, 9, 8, 1],

[47, 42, 39, 34, 31, 26, 23, 18, 15, 10, 7, 2],

[46, 43, 38, 35, 30, 27, 22, 19, 14, 11, 6, 3],

[45, 44, 37, 36, 29, 28, 21, 20, 13, 12, 5, 4],

]

TREATMENTS = [

["D3V3", "D1V1", "D2V2", "D4V3", "D3V2", "D1V3", "D1V1", "D3V2", "D2V3", "D2V1", "D2V3", "D4V2"],

["D4V1", "D1V2", "D2V3", "D1V1", "D4V2", "D2V1", "D1V3", "D4V2", "D3V1", "D3V3", "D1V1", "D2V2"],

["D4V3", "D4V1", "D1V3", "D2V2", "D1V2", "D2V3", "D2V1", "D2V2", "D4V1", "D3V2", "D4V3", "D1V3"],

["D2V2", "D3V2", "D3V1", "D3V1", "D3V3", "D4V1", "D3V3", "D4V3", "D1V2", "D4V1", "D1V2", "D3V1"],

]

BLOCKS = ["R1"] * 3 + ["R2"] * 3 + ["R3"] * 3 + ["CONTROL"] * 3   # by column

def build_layout_table():

rows = []

for r in range(C.N_ROWS):

for c in range(C.N_COLS):

t = TREATMENTS[r][c]

rows.append(dict(plot_id=PLOT_IDS[r][c], row=r + 1, col=c + 1, treatment=t,

inoc_age=t[:2], variety=t[2:], block=BLOCKS[c],

inoculated=BLOCKS[c] != "CONTROL"))

return pd.DataFrame(rows).sort_values("plot_id").reset_index(drop=True)

def check_layout(layout):

"""Every replication block should contain each of the 12 treatments exactly once."""

problems = []

for block, g in layout.groupby("block"):

counts = g["treatment"].value_counts()

dup = counts[counts > 1].index.tolist()

missing = sorted({f"D{d}V{v}" for d in range(1, 5) for v in range(1, 4)} - set(g["treatment"]))

if dup or missing:

problems.append(f"{block}: repeated {dup}, missing {missing}")

return problems

def row_top_offset(r):

"""Distance (m) from the field's north edge to the top of plot row r (1-based)."""

channels_above = (1 if r >= 2 else 0) + (1 if r >= 4 else 0)

return (r - 1) * C.PLOT_LENGTH_M + channels_above * C.CHANNEL_M

def build_plot_polygons(layout):

gap = (C.FIELD_WIDTH_M - C.N_COLS * C.PLOT_WIDTH_M) / (C.N_COLS - 1)

geoms = []

for _, p in layout.iterrows():

x0 = (p.col - 1) * (C.PLOT_WIDTH_M + gap)

y0 = -row_top_offset(p.row)

poly = Polygon([(x0, y0), (x0 + C.PLOT_WIDTH_M, y0),

(x0 + C.PLOT_WIDTH_M, y0 - C.PLOT_LENGTH_M), (x0, y0 - C.PLOT_LENGTH_M)])

poly = affinity.rotate(poly, C.FIELD_ROTATION_DEG, origin=(0, 0))

geoms.append(affinity.translate(poly, C.FIELD_ORIGIN_E, C.FIELD_ORIGIN_N))

gdf = gpd.GeoDataFrame(layout.copy(), geometry=geoms, crs=C.CRS)

gdf["geometry_inner"] = gdf.geometry.buffer(-C.PLOT_INSET_M, join_style="mitre")

return gdf

def di_to_class(di):

return pd.cut(di, bins=C.DI_CLASS_BINS, labels=range(len(C.CLASS_NAMES))).astype(int)

def load_ground_truth(layout):

gt = pd.read_csv(C.GROUND_TRUTH_CSV, dtype={"date": str})

gt["severity_class"] = di_to_class(gt["DI"])

gt["diseased"] = (gt["severity_class"] > 0).astype(int)

gt = gt.merge(layout[["plot_id", "block", "treatment", "inoc_age", "variety", "inoculated"]],

on="plot_id", how="left")

bad = gt[(~gt["inoculated"]) & (gt["DI"] > 0)]

if len(bad):

print(f"WARNING: {len(bad)} control plot-dates have DI > 0 (natural infection?)")

return gt

if __name__ == "__main__":

C.ROOT.mkdir(parents=True, exist_ok=True)

layout = build_layout_table()

for msg in check_layout(layout):

print("LAYOUT CHECK:", msg)

layout.to_csv(C.LAYOUT_CSV, index=False)

plots = build_plot_polygons(layout)

plots.set_geometry("geometry").drop(columns="geometry_inner").to_file(C.PLOTS_FILE, layer="plots")

print(f"Wrote {len(plots)} plot polygons to {C.PLOTS_FILE}")

if C.GROUND_TRUTH_CSV.exists():

gt = load_ground_truth(layout)

print(gt.groupby(["date", "severity_class"]).size().unstack(fill_value=0))

# 4. Shared helpers (common.py)

Functions used by several steps: reading and writing GeoTIFFs, reading hyperspectral wavelengths from an ENVI header, finding the band nearest to a wavelength, and a division that returns NaN instead of infinity.

File: rustwatch_pipeline/common.py

"""Shared helpers: raster input/output, wavelengths, band lookup."""

import re

import numpy as np

import rasterio

import config as C

def read_raster(path):

"""Return (array[bands, rows, cols] as float32, rasterio profile)."""

with rasterio.open(path) as src:

arr = src.read().astype("float32")

prof = src.profile.copy()

if src.nodata is not None:

arr[arr == src.nodata] = np.nan

return arr, prof

def write_raster(path, arr, prof, dtype="float32", nodata=np.nan):

path.parent.mkdir(parents=True, exist_ok=True)

arr = np.asarray(arr)

if arr.ndim == 2:

arr = arr[None]

p = prof.copy()

p.update(driver="GTiff", count=arr.shape[0], dtype=dtype, nodata=nodata,

compress="deflate", tiled=True, blockxsize=256, blockysize=256, BIGTIFF="IF_SAFER")

p.pop("interleave", None)

with rasterio.open(path, "w", **p) as dst:

dst.write(arr.astype(dtype))

def read_hs_wavelengths(path):

"""Wavelengths (nm) from an ENVI header next to the cube, else from config.HS_WAVELENGTH_FILE."""

hdr = path.with_suffix(".hdr")

if hdr.exists():

txt = hdr.read_text()

m = re.search(r"wavelength\s*=\s*\{([^}]*)\}", txt, re.IGNORECASE)

if m:

wl = np.array([float(v) for v in m.group(1).replace("\n", " ").split(",") if v.strip()])

return wl * 1000 if wl.max() < 10 else wl     # micrometres -> nm

return np.loadtxt(C.HS_WAVELENGTH_FILE)

def band_index(wavelengths, target_nm, tol=15):

"""Index of the band nearest to target_nm, or None if none within tol nm."""

wavelengths = np.asarray(wavelengths)

i = int(np.argmin(np.abs(wavelengths - target_nm)))

return i if abs(wavelengths[i] - target_nm) <= tol else None

def ms_band(arr, name):

return arr[list(C.MS_BANDS).index(name)]

def safe_div(a, b):

"""Element-wise a / b with inf and NaN results set to NaN (works for arrays and scalars)."""

with np.errstate(divide="ignore", invalid="ignore"):

out = np.true_divide(a, b)

out = np.where(np.isfinite(out), out, np.nan)

return out if np.ndim(out) else float(out)

# 5. Step 2: Pre-processing and co-registration (s02_preprocess.py)

## 5.1 Before Python

- Orthomosaics. Stitch each sensor and date in Pix4D, Agisoft Metashape or Headwall SpectralView, georeferenced with the 12 RTK ground-control panels, and export GeoTIFF (or ENVI for the hyperspectral cube) in the project CRS.

- Reflectance. For MS and HS, use the calibration panel and the irradiance (DLS) sensor in the stitching software to export reflectance. If you export digital numbers instead, provide calibration_panels.csv and the code applies the empirical line.

- Thermal. Export radiometric temperature from FLIR Tools (°C or Kelvin). If you export raw counts, check the scale factor (counts_to_kelvin, 0.04 K per count for TLinear high resolution) against FLIR Tools.

- Hyperspectral VNIR and SWIR are usually separate cubes at different resolutions. Resample SWIR onto the VNIR grid and stack them into one cube (one wavelength list), or treat them as two sensors (add “HSS” to SENSORS).

## 5.2 What the code does

- Converts units: scaled integers → reflectance 0–1 (÷255, ÷10,000 or ÷65,535, chosen from the data range and printed); Kelvin or radiometric counts → °C.

- Applies the two-panel empirical-line correction per band if panel values are given.

- Removes hyperspectral bands in HS_BAD_RANGES and smooths each continuous spectral segment with a Savitzky–Golay filter (window 11, order 2).

- Co-registers each sensor to the multispectral image: a band with strong canopy/soil contrast is resampled onto the MS grid, edge images are compared by phase correlation (sub-pixel), and the measured shift is applied to the image’s geotransform (no resampling of the data). Shifts larger than 1 m are reported and not applied; that usually means a GCP problem.

- Writes analysis-ready rasters to 02_preprocessed/ and the shifts to coregistration_log.csv.

Check. Open coregistration_log.csv: shifts should be a few centimetres to a few decimetres. On the synthetic test data, the deliberate 24 cm offset of the thermal images was detected as 6 MS pixels (6 × 4 cm) on every date.

File: rustwatch_pipeline/s02_preprocess.py

"""Step 2 - Image pre-processing.

Assumes each orthomosaic was already stitched (Pix4D / Agisoft / Headwall SpectralView)

and georeferenced with the 12 RTK ground-control points. This step then:

2a  converts to physical units (reflectance 0-1, temperature in deg C),

2b  applies the empirical-line correction if calibration-panel values are supplied,

2c  removes noisy hyperspectral bands and smooths spectra (Savitzky-Golay),

2d  co-registers every sensor to the reference sensor (sub-pixel shift),

2e  writes analysis-ready GeoTIFFs.

"""

import numpy as np

import pandas as pd

import rasterio

from rasterio.warp import reproject, Resampling

from rasterio.transform import Affine

from scipy.signal import savgol_filter

from skimage.registration import phase_cross_correlation

from skimage.filters import sobel

import config as C

from common import read_raster, write_raster, read_hs_wavelengths, band_index, ms_band

PANEL_CSV = C.ROOT / "calibration_panels.csv"   # optional: sensor,date,band,dn_dark,dn_bright,refl_dark,refl_bright

# ------------------------------------------------------------------ 2a units

def to_reflectance(arr, sensor):

"""Scale digital numbers to 0-1 reflectance if the orthomosaic is stored as integers."""

p99 = np.nanpercentile(arr, 99)

if p99 > 1.5:

scale = 255.0 if p99 <= 255 else (65535.0 if p99 > 20000 else 10000.0)

print(f"  {sensor}: values look scaled (p99={p99:.0f}); dividing by {scale:.0f}")

arr = arr / scale

return np.clip(arr, 0, 1.5)

def to_celsius(arr, counts_to_kelvin=0.04):

"""Thermal: accept deg C, Kelvin, or FLIR radiometric counts (TLinear, 0.04 K per count)."""

med = np.nanmedian(arr)

if med > 1000:

print("  TIR: radiometric counts -> deg C (check the scale factor in FLIR Tools)")

return arr * counts_to_kelvin - 273.15

if med > 200:

print("  TIR: Kelvin -> deg C")

return arr - 273.15

return arr

# ------------------------------------------------------------------ 2b empirical line

def empirical_line(arr, sensor, date):

"""Two-panel empirical line per band: refl = gain * DN + offset."""

if not PANEL_CSV.exists():

return arr

cal = pd.read_csv(PANEL_CSV, dtype={"date": str})

cal = cal[(cal.sensor == sensor) & (cal.date == date)]

for _, r in cal.iterrows():

b = int(r.band)

gain = (r.refl_bright - r.refl_dark) / (r.dn_bright - r.dn_dark)

arr[b] = gain * arr[b] + (r.refl_dark - gain * r.dn_dark)

if len(cal):

print(f"  {sensor} {date}: empirical-line correction applied to {len(cal)} bands")

return arr

# ------------------------------------------------------------------ 2c hyperspectral cleaning

def clean_hyperspectral(cube, wavelengths):

keep = np.ones(len(wavelengths), bool)

for lo, hi in C.HS_BAD_RANGES:

keep &= ~((wavelengths >= lo) & (wavelengths <= hi))

cube, wavelengths = cube[keep], wavelengths[keep]

win = min(C.SG_WINDOW, len(wavelengths) - (1 - len(wavelengths) % 2))

if win > C.SG_POLY + 1:

# smooth each contiguous spectral segment separately so gaps are not bridged

breaks = np.where(np.diff(wavelengths) > 3 * np.median(np.diff(wavelengths)))[0] + 1

for seg in np.split(np.arange(len(wavelengths)), breaks):

w = min(win, len(seg) if len(seg) % 2 else len(seg) - 1)

if w > C.SG_POLY + 1:

block = cube[seg]

nan = np.isnan(block)

block = savgol_filter(np.nan_to_num(block), w, C.SG_POLY, axis=0)

block[nan] = np.nan

cube[seg] = block

return cube, wavelengths

# ------------------------------------------------------------------ 2d co-registration

def registration_band(arr, sensor, wavelengths=None):

"""A band with clear canopy/soil contrast for each sensor."""

if sensor == "MS":

return ms_band(arr, "NIR")

if sensor == "HS":

return arr[band_index(wavelengths, 800, tol=60)]

if sensor == "RGB":

return arr[1]

return -arr[0]          # thermal: canopy is cool, soil warm -> invert for similar contrast

def resample_to(src_arr, src_prof, dst_prof, method=Resampling.average):

dst = np.full((src_arr.shape[0], dst_prof["height"], dst_prof["width"]), np.nan, "float32")

for b in range(src_arr.shape[0]):

reproject(src_arr[b], dst[b], src_transform=src_prof["transform"], src_crs=src_prof["crs"],

dst_transform=dst_prof["transform"], dst_crs=dst_prof["crs"],

src_nodata=np.nan, dst_nodata=np.nan, resampling=method)

return dst

def estimate_shift(moving, reference):

"""Sub-pixel (row, col) shift that aligns `moving` to `reference`, using edge images."""

def prep(x):

x = np.nan_to_num((x - np.nanmean(x)) / (np.nanstd(x) + 1e-9))

return sobel(x)

shift, error, _ = phase_cross_correlation(prep(reference), prep(moving), upsample_factor=20)

return shift, error

def apply_shift(prof, shift_rows, shift_cols, ref_prof):

"""Translate the geotransform by a shift measured in reference-grid pixels."""

t, rt = prof["transform"], ref_prof["transform"]

new = Affine(t.a, t.b, t.c + shift_cols * rt.a, t.d, t.e, t.f + shift_rows * rt.e)

out = prof.copy()

out["transform"] = new

return out

# ------------------------------------------------------------------ main

def preprocess_all(max_shift_m=1.0):

log = []

for date in C.DATES:

ref_arr, ref_prof = None, None

for sensor in [C.REFERENCE_SENSOR] + [s for s in C.SENSORS if s != C.REFERENCE_SENSOR]:

path = C.raw_path(sensor, date)

if not path.exists():

print(f"missing {path}")

continue

arr, prof = read_raster(path)

wl = None

if sensor == "TIR":

arr = to_celsius(arr)

else:

arr = empirical_line(arr, sensor, date)

arr = to_reflectance(arr, sensor)

if sensor == "HS":

wl = read_hs_wavelengths(path)

arr, wl = clean_hyperspectral(arr, wl)

(C.PRE / "HS").mkdir(parents=True, exist_ok=True)

np.savetxt(C.PRE / "HS" / "wavelengths.txt", wl, fmt="%.2f")

if sensor == C.REFERENCE_SENSOR:

ref_arr, ref_prof = arr, prof

shift = (0.0, 0.0)

else:

mov = resample_to(registration_band(arr, sensor, wl)[None], prof, ref_prof)[0]

shift, err = estimate_shift(mov, registration_band(ref_arr, C.REFERENCE_SENSOR))

shift_m = np.hypot(shift[0] * ref_prof["transform"].e, shift[1] * ref_prof["transform"].a)

if shift_m > max_shift_m:

print(f"  WARNING {sensor} {date}: shift {shift_m:.2f} m exceeds {max_shift_m} m; not applied - check GCPs")

shift = (0.0, 0.0)

else:

prof = apply_shift(prof, shift[0], shift[1], ref_prof)

log.append(dict(sensor=sensor, date=date, shift_rows=shift[0], shift_cols=shift[1]))

write_raster(C.PRE / sensor / f"{sensor}_{date}.tif", arr, prof)

print(f"{sensor} {date}: {arr.shape} written; shift (ref px) = {np.round(shift, 2)}")

pd.DataFrame(log).to_csv(C.PRE / "coregistration_log.csv", index=False)

if __name__ == "__main__":

preprocess_all()

# 6. Step 3: Vegetation masking, soil and shadow removal (s03_masking.py)

| Sensor | Vegetation rule | Removed |
| --- | --- | --- |
| MS | NDVI > Otsu threshold (or fixed NDVI_MIN); K-means alternative | Soil (low NDVI), shadow (darkest 5 % NIR) |
| HS | NDVI(800, 670) > Otsu threshold | Soil, shadow |
| RGB | Excess Green (2g − r − b) > Otsu threshold | Soil, dark pixels |
| TIR | MS vegetation mask of the same date, resampled to the thermal grid (≥ 80 % canopy per thermal pixel), minus pixels warmer than the canopy/soil Otsu split | Soil and mixed warm pixels |

Why thermal uses the MS mask. A thermal image cannot separate soil from canopy reliably on its own, and warm diseased canopy can look like soil. The co-registered multispectral mask defines where the canopy is.

Check. Open a mask over its image in QGIS for one healthy and one severe plot. Severe disease lowers NDVI towards soil values; if severely diseased canopy is being removed, set a fixed low threshold (for example NDVI_MIN = 0.25) instead of Otsu. Step 5 also has a fallback for plots left with almost no vegetation pixels.

File: rustwatch_pipeline/s03_masking.py

"""Step 3 - Vegetation masking, soil removal and shadow removal.

MS and HS : NDVI > Otsu threshold, minus the darkest NIR pixels (shadow).

RGB       : Excess Green (ExG) > Otsu threshold, minus very dark pixels.

TIR       : no spectral information, so the MS vegetation mask of the same date is

resampled onto the thermal grid, then pixels warmer than the

canopy/soil Otsu split are removed (mixed soil pixels).

Optional  : K-means clustering instead of a threshold (method="kmeans").

"""

import numpy as np

from skimage.filters import threshold_otsu

from skimage.morphology import binary_opening, disk

from sklearn.cluster import KMeans

from rasterio.warp import Resampling

import config as C

from common import read_raster, write_raster, band_index, ms_band, safe_div

from s02_preprocess import resample_to

def ndvi_ms(arr):

return safe_div(ms_band(arr, "NIR") - ms_band(arr, "R"), ms_band(arr, "NIR") + ms_band(arr, "R"))

def ndvi_hs(arr, wl):

r, n = arr[band_index(wl, 670)], arr[band_index(wl, 800, tol=40)]

return safe_div(n - r, n + r)

def excess_green(rgb):

s = np.nansum(rgb, axis=0) + 1e-9

r, g, b = rgb[0] / s, rgb[1] / s, rgb[2] / s

return 2 * g - r - b

def otsu_mask(index, fixed=None):

v = index[np.isfinite(index)]

t = fixed if fixed is not None else threshold_otsu(v)

return np.nan_to_num(index, nan=-9) > t, t

def kmeans_mask(features, veg_feature=0, k=3, seed=C.RANDOM_STATE):

"""features: list of 2-D arrays. Cluster with the highest mean of features[veg_feature] = vegetation."""

X = np.stack([f.ravel() for f in features], 1)

ok = np.all(np.isfinite(X), 1)

sample = np.random.default_rng(seed).choice(np.where(ok)[0], min(50000, ok.sum()), replace=False)

km = KMeans(k, n_init=10, random_state=seed).fit(X[sample])

lab = np.full(len(X), -1)

lab[ok] = km.predict(X[ok])

veg = int(np.argmax(km.cluster_centers_[:, veg_feature]))

return (lab == veg).reshape(features[0].shape)

def shadow_mask(nir):

return nir < np.nanpercentile(nir, C.SHADOW_NIR_PERCENTILE)

def clean(mask, radius=1):

return binary_opening(mask, disk(radius)) if radius else mask

def mask_date(date, method="otsu"):

out = {}

ms, ms_prof = read_raster(C.PRE / "MS" / f"MS_{date}.tif")

ndvi = ndvi_ms(ms)

if method == "kmeans":

veg = kmeans_mask([ndvi, ms_band(ms, "NIR")])

t = np.nan

else:

veg, t = otsu_mask(ndvi, C.NDVI_MIN)

veg = clean(veg & ~shadow_mask(ms_band(ms, "NIR")))

out["MS"] = (veg, ms_prof, t)

hs_path = C.PRE / "HS" / f"HS_{date}.tif"

if hs_path.exists():

hs, hs_prof = read_raster(hs_path)

wl = np.loadtxt(C.PRE / "HS" / "wavelengths.txt")

veg_h, t_h = otsu_mask(ndvi_hs(hs, wl), C.NDVI_MIN)

nir = hs[band_index(wl, 800, tol=40)]

out["HS"] = (clean(veg_h & ~shadow_mask(nir)), hs_prof, t_h)

rgb_path = C.PRE / "RGB" / f"RGB_{date}.tif"

if rgb_path.exists():

rgb, rgb_prof = read_raster(rgb_path)

veg_r, t_r = otsu_mask(excess_green(rgb), C.RGB_EXG_MIN)

bright = np.nansum(rgb, 0)

dark = bright < np.nanpercentile(bright, C.SHADOW_NIR_PERCENTILE)

out["RGB"] = (clean(veg_r & ~dark, radius=2), rgb_prof, t_r)

tir_path = C.PRE / "TIR" / f"TIR_{date}.tif"

if tir_path.exists():

tir, tir_prof = read_raster(tir_path)

ms_on_tir = resample_to(out["MS"][0][None].astype("float32"), ms_prof, tir_prof, Resampling.average)[0]

veg_t = np.nan_to_num(ms_on_tir) > 0.8                      # mostly-canopy thermal pixels

warm, t_t = otsu_mask(tir[0])                               # soil is warmer than canopy

out["TIR"] = (veg_t & ~warm, tir_prof, t_t)

for sensor, (m, prof, t) in out.items():

write_raster(C.MASK / sensor / f"{sensor}_{date}_veg.tif", m.astype("uint8"), prof, dtype="uint8", nodata=255)

print(f"{sensor} {date}: threshold={t:.3f}  vegetation fraction={m.mean():.2f}")

return out

if __name__ == "__main__":

for d in C.DATES:

mask_date(d)

# 7. Step 4: Endmember extraction and spectral unmixing (s04_endmembers.py)

Why. At 9.7 cm (VNIR) and 36 cm (SWIR) a hyperspectral pixel mixes canopy, soil, shadow and pustule-affected tissue. Unmixing estimates the fraction of each pure material in every pixel, so the plot-mean fraction of “diseased canopy” becomes a feature that does not depend on a hard mask.

- Sampling. Pixels from all four dates inside the plots are sampled, and one set of endmembers is extracted for all dates so that abundances are comparable over time.

- VCA (Vertex Component Analysis; Nascimento & Bioucas-Dias, 2005) finds the p purest pixels as the vertices of the data simplex. Because VCA is randomised, it runs 10 times and keeps the run with the largest simplex volume.

- Naming. Reference spectra are taken from the data: control-plot vegetation (healthy), vegetation of the 10 % most severe plots (diseased), bright non-vegetation pixels (soil) and the darkest 2 % (shadow). Each endmember is matched to a reference by minimum spectral angle (Hungarian assignment).

- FCLS unmixing (Heinz & Chang, 2001): abundances are non-negative and sum to one; solved by non-negative least squares with an appended sum-to-one row.

- Writes HS_endmembers.csv, HS_endmembers.png and one abundance raster per date.

Check. Look at HS_endmembers.png: soil should rise smoothly with wavelength; healthy canopy should show a green peak, deep red absorption and a high NIR plateau; diseased canopy a shallower red absorption and lower NIR. If two endmembers look alike, reduce N_ENDMEMBERS to 3. The same function works for MS with up to 5 endmembers (run_unmixing("MS")).

File: rustwatch_pipeline/s04_endmembers.py

"""Step 4 - Endmember extraction (VCA) and spectral unmixing (FCLS).

4a  sample pixels from all dates inside the plots (one endmember set for all dates,

so abundances are comparable over time),

4b  extract p endmembers with Vertex Component Analysis (Nascimento & Bioucas-Dias, 2005),

4c  name the endmembers (soil, healthy canopy, diseased canopy, shadow) by spectral

angle to reference spectra taken from the data (control plots = healthy,

most severe plots = diseased, bright non-vegetation = soil, darkest pixels = shadow),

4d  unmix every plot pixel with Fully Constrained Least Squares (abundances >= 0, sum = 1),

4e  write abundance rasters; plot means become features in Step 5.

Works for HS (default) and, with <= 5 endmembers, for MS.

"""

import numpy as np

import pandas as pd

import geopandas as gpd

from rasterio.features import rasterize

from scipy.optimize import nnls, linear_sum_assignment

import matplotlib

matplotlib.use("Agg")

import matplotlib.pyplot as plt

import config as C

from common import read_raster, write_raster

NAMES = ["soil", "healthy", "diseased", "shadow"]

def plot_id_raster(plots, prof):

shapes = [(g, int(pid)) for g, pid in zip(plots.geometry, plots.plot_id)]

return rasterize(shapes, out_shape=(prof["height"], prof["width"]), transform=prof["transform"], fill=0, dtype="int32")

def vca(Y, p, seed=0):

"""Vertex Component Analysis. Y: (bands, pixels). Returns endmembers (bands, p) and pixel indices."""

rng = np.random.default_rng(seed)

L, N = Y.shape

U, _, _ = np.linalg.svd(Y @ Y.T / N)

X = U[:, :p].T @ Y                                   # project onto signal subspace

u = X.mean(1, keepdims=True)

Yp = X / (u.T @ X)                                   # projective projection

A = np.zeros((p, p)); A[-1, 0] = 1

idx = []

for i in range(p):

w = rng.standard_normal((p, 1))

f = w - A @ np.linalg.pinv(A) @ w

f /= np.linalg.norm(f)

k = int(np.argmax(np.abs(f.T @ Yp)))

A[:, i] = Yp[:, k]

idx.append(k)

return Y[:, idx], np.array(idx)

def simplex_volume(E):

Z = E - E.mean(1, keepdims=True)

U, _, _ = np.linalg.svd(Z, full_matrices=False)

P = U[:, :E.shape[1] - 1].T @ E

return abs(np.linalg.det(P[:, 1:] - P[:, [0]]))

def best_vca(Y, p, runs=10):

"""VCA is randomised; keep the run whose endmembers span the largest simplex."""

return max((vca(Y, p, s) for s in range(runs)), key=lambda r: simplex_volume(r[0]))

def spectral_angle(a, b):

return np.arccos(np.clip(a @ b / (np.linalg.norm(a) * np.linalg.norm(b) + 1e-12), -1, 1))

def name_endmembers(E, refs):

names = list(refs)

cost = np.array([[spectral_angle(E[:, i], refs[n]) for n in names] for i in range(E.shape[1])])

rows, cols = linear_sum_assignment(cost)

labels = [f"em{i}" for i in range(E.shape[1])]

for r, c in zip(rows, cols):

labels[r] = names[c]

return labels

def fcls(E, Y, delta=None):

"""Fully constrained least squares via NNLS with an appended sum-to-one row."""

delta = delta or 10 * np.nanmax(E)

Ea = np.vstack([np.full((1, E.shape[1]), delta), E])

out = np.zeros((E.shape[1], Y.shape[1]), "float32")

for j in range(Y.shape[1]):

out[:, j] = nnls(Ea, np.concatenate([[delta], Y[:, j]]))[0]

return out

def reference_spectra(cube, veg, pid, gt_date):

healthy_ids = gt_date.loc[gt_date.DI == 0, "plot_id"].values

severe_ids = gt_date.loc[gt_date.DI >= gt_date.DI.quantile(0.9), "plot_id"].values

bright = np.nanmean(cube, 0)

inside = pid > 0

sel = {

"healthy": veg & np.isin(pid, healthy_ids),

"diseased": veg & np.isin(pid, severe_ids),

"soil": inside & ~veg & (bright > np.nanpercentile(bright[inside], 50)),

"shadow": inside & (bright < np.nanpercentile(bright[inside], 2)),

}

return {k: np.nanmean(cube[:, m], 1) for k, m in sel.items() if m.sum() > 10}

def run_unmixing(sensor="HS", p=C.N_ENDMEMBERS):

plots = gpd.read_file(C.PLOTS_FILE)

gt = pd.read_csv(C.GROUND_TRUTH_CSV, dtype={"date": str})

rng = np.random.default_rng(C.RANDOM_STATE)

samples, refs_all, cache = [], [], {}

for date in C.DATES:

cube, prof = read_raster(C.PRE / sensor / f"{sensor}_{date}.tif")

veg = read_raster(C.MASK / sensor / f"{sensor}_{date}_veg.tif")[0][0] == 1

pid = plot_id_raster(plots, prof)

cache[date] = (prof, pid)

Y = cube[:, pid > 0]

Y = Y[:, np.all(np.isfinite(Y), 0)]

samples.append(Y[:, rng.choice(Y.shape[1], min(C.UNMIX_SAMPLE // len(C.DATES), Y.shape[1]), replace=False)])

refs_all.append(reference_spectra(cube, veg, pid, gt[gt.date == date]))

Ysamp = np.hstack(samples)

E, _ = best_vca(Ysamp, p)

refs = {k: np.nanmean([r[k] for r in refs_all if k in r], 0) for k in NAMES if any(k in r for r in refs_all)}

labels = name_endmembers(E, refs)

C.UNMIX.mkdir(parents=True, exist_ok=True)

pd.DataFrame(E, columns=labels).to_csv(C.UNMIX / f"{sensor}_endmembers.csv", index=False)

fig, ax = plt.subplots(figsize=(7, 4))

wl = np.loadtxt(C.PRE / "HS" / "wavelengths.txt") if sensor == "HS" else np.array(list(C.MS_BANDS.values()))

for i, n in enumerate(labels):

ax.plot(wl, E[:, i], label=n)

ax.set_xlabel("Wavelength (nm)"); ax.set_ylabel("Reflectance"); ax.legend(); fig.tight_layout()

fig.savefig(C.UNMIX / f"{sensor}_endmembers.png", dpi=150); plt.close(fig)

print(f"{sensor} endmembers: {labels}")

for date in C.DATES:

cube = read_raster(C.PRE / sensor / f"{sensor}_{date}.tif")[0]

prof, pid = cache[date]

inside = (pid > 0) & np.all(np.isfinite(cube), 0)

ab = np.full((p, *pid.shape), np.nan, "float32")

ab[:, inside] = fcls(E, cube[:, inside])

p2 = prof.copy()

write_raster(C.UNMIX / f"{sensor}_{date}_abundance.tif", ab, p2)

print(f"{sensor} {date}: unmixed {inside.sum()} pixels")

return labels

if __name__ == "__main__":

run_unmixing("HS")

# 8. Step 5: Plot-wise feature extraction (s05_features.py)

Each plot polygon is shrunk inward by 0.4 m; only vegetation pixels inside it are summarised. Reading is done window by window, so large orthomosaics do not have to fit in memory.

| Sensor | Features (prefix) |
| --- | --- |
| MS | Band reflectance (B, G, R, RE, NIR); NDVI, GNDVI, NDRE, CIre, CIg, MTCI, TCARI, SAVI, OSAVI, MSR, SR, PSRI; NDVI standard deviation; canopy cover (MS_) |
| HS | Reflectance of every retained band (HS_R<wavelength>); PRI, MCARI, TCARI, PSRI, SIPI, ARI, NDWI, WI, NDVI705, HI; first-derivative maxima, positions and sums in the blue, yellow and red edges; SDr/SDb, SDr/SDy; red-edge position; continuum-removed depth and area at 670, 970 and 1200 nm; endmember abundances and the diseased share (HS_) |
| RGB | ExG, ExR, VARI, GLI, NGRDI; 8 GLCM metrics (mean, variance, homogeneity, contrast, dissimilarity, entropy, ASM, correlation) × 3 bands, averaged over 4 directions (RGB_) |
| TIR | Canopy temperature CT; CTD = CT − AT; CTR = CT / AT; NRCT = (CT − CTmin)/(CTmax − CTmin) with field-wide robust min/max; temperature mean, SD, 10th and 90th percentiles; 8 GLCM metrics (TIR_) |
| All | Canopy cover (vegetation pixels / plot pixels) |

GLCM with a mask. Soil pixels are coded as grey level 0 and the co-occurrence pairs involving level 0 are dropped, so texture describes the canopy only. Low-cover fallback. If fewer than 20 vegetation pixels remain in a plot (this happened for the most severe synthetic plots at 20 cm hyperspectral resolution), all plot pixels are used and the plot is listed on screen; canopy cover itself is kept as a feature.

Output. 05_features/features_<sensor>.csv, one row per plot and date (192 rows).

File: rustwatch_pipeline/s05_features.py

"""Step 5 - Plot-wise feature extraction.

For every sensor, date and plot (polygon shrunk inward by PLOT_INSET_M) the vegetation

pixels are summarised into one row of features:

MS  : band reflectance, 12 vegetation indices, canopy cover

HS  : mean spectrum (every retained band), first-derivative edge features, red-edge

position, continuum-removed absorption features, narrow-band indices,

endmember abundances

RGB : colour indices and GLCM texture (8 metrics x 3 bands)

TIR : canopy temperature statistics, CTD, CTR, NRCT and GLCM texture

Output: FEAT/features_<sensor>.csv  (one row per plot_id x date)

"""

import numpy as np

import pandas as pd

import geopandas as gpd

import rasterio

from rasterio.features import geometry_mask

from rasterio.windows import from_bounds

from skimage.feature import graycomatrix

import config as C

from common import read_hs_wavelengths, band_index, safe_div

# ------------------------------------------------------------------ helpers

def plot_pixels(src, geom, extra_mask_src=None):

"""Read the window around one plot; return (array[bands,r,c], plot_mask, veg_mask)."""

win = from_bounds(*geom.bounds, transform=src.transform).round_offsets().round_lengths()

arr = src.read(window=win).astype("float32")

tr = src.window_transform(win)

inside = ~geometry_mask([geom], out_shape=arr.shape[1:], transform=tr)

veg = inside.copy()

if extra_mask_src is not None:

veg &= extra_mask_src.read(1, window=win, boundless=True, fill_value=0) == 1

if src.nodata is not None and not np.isnan(src.nodata):

arr[arr == src.nodata] = np.nan

return arr, inside, veg

def glcm_features(img, mask, levels=32, prefix=""):

"""Eight Haralick metrics averaged over 0, 45, 90, 135 degrees, using only masked pixels."""

names = ["mean", "variance", "homogeneity", "contrast", "dissimilarity", "entropy", "asm", "correlation"]

vals = img[mask & np.isfinite(img)]

if vals.size < 50:

return {f"{prefix}glcm_{n}": np.nan for n in names}

lo, hi = np.percentile(vals, [1, 99])

q = np.clip(((img - lo) / (hi - lo + 1e-9) * (levels - 2)), 0, levels - 2)

q = np.nan_to_num(q).astype(np.uint8) + 1          # 1..levels-1 for vegetation

q[~mask] = 0                                       # 0 = excluded (soil, outside plot)

P = graycomatrix(q, [1], [0, np.pi / 4, np.pi / 2, 3 * np.pi / 4], levels=levels, symmetric=True)

P = P[1:, 1:, 0, :].astype(float)                  # drop pairs touching excluded pixels

i, j = np.indices(P.shape[:2])

acc = {n: [] for n in names}

for a in range(P.shape[2]):

p = P[:, :, a]

if p.sum() == 0:

continue

p = p / p.sum()

mi, mj = (i * p).sum(), (j * p).sum()

vi, vj = ((i - mi) ** 2 * p).sum(), ((j - mj) ** 2 * p).sum()

nz = p[p > 0]

acc["mean"].append(mi); acc["variance"].append(vi)

acc["homogeneity"].append((p / (1 + (i - j) ** 2)).sum())

acc["contrast"].append(((i - j) ** 2 * p).sum())

acc["dissimilarity"].append((np.abs(i - j) * p).sum())

acc["entropy"].append(-(nz * np.log(nz)).sum())

acc["asm"].append((p ** 2).sum())

acc["correlation"].append(((i - mi) * (j - mj) * p).sum() / np.sqrt(vi * vj + 1e-12))

return {f"{prefix}glcm_{n}": float(np.mean(v)) if v else np.nan for n, v in acc.items()}

def stats(x, prefix):

x = x[np.isfinite(x)]

if x.size == 0:

return {f"{prefix}_{k}": np.nan for k in ["mean", "std", "p10", "p90"]}

return {f"{prefix}_mean": x.mean(), f"{prefix}_std": x.std(),

f"{prefix}_p10": np.percentile(x, 10), f"{prefix}_p90": np.percentile(x, 90)}

# ------------------------------------------------------------------ MS

def ms_indices(b):

B, G, R, RE, N = b["B"], b["G"], b["R"], b["RE"], b["NIR"]

return {

"NDVI": safe_div(N - R, N + R), "GNDVI": safe_div(N - G, N + G), "NDRE": safe_div(N - RE, N + RE),

"CIre": safe_div(N, RE) - 1, "CIg": safe_div(N, G) - 1, "MTCI": safe_div(N - RE, RE - R),

"TCARI": 3 * ((RE - R) - 0.2 * (RE - G) * safe_div(RE, R)),

"SAVI": 1.5 * safe_div(N - R, N + R + 0.5), "OSAVI": safe_div(N - R, N + R + 0.16),

"MSR": safe_div(safe_div(N, R) - 1, np.sqrt(np.clip(safe_div(N, R), 0, None)) + 1),

"SR": safe_div(N, R), "PSRI": safe_div(R - B, N),

}

def ms_features(arr, inside, veg):

b = {name: arr[i] for i, name in enumerate(C.MS_BANDS)}

f = {f"MS_{k}": np.nanmean(v[veg]) for k, v in b.items()}

for k, v in ms_indices(b).items():

f[f"MS_{k}"] = np.nanmean(v[veg])

f["MS_NDVI_std"] = np.nanstd(ms_indices(b)["NDVI"][veg])

return f

# ------------------------------------------------------------------ HS

HS_VIS = {

"PRI": lambda r: safe_div(r(531) - r(570), r(531) + r(570)),

"MCARI": lambda r: ((r(700) - r(670)) - 0.2 * (r(700) - r(550))) * safe_div(r(700), r(670)),

"TCARI": lambda r: 3 * ((r(700) - r(670)) - 0.2 * (r(700) - r(550)) * safe_div(r(700), r(670))),

"PSRI": lambda r: safe_div(r(680) - r(500), r(750)),

"SIPI": lambda r: safe_div(r(800) - r(445), r(800) - r(680)),

"ARI": lambda r: safe_div(1.0, r(550)) - safe_div(1.0, r(700)),

"NDWI": lambda r: safe_div(r(860) - r(1240), r(860) + r(1240)),

"WI": lambda r: safe_div(r(900), r(970)),

"NDVI705": lambda r: safe_div(r(750) - r(705), r(750) + r(705)),

"HI": lambda r: safe_div(r(534) - r(698), r(534) + r(698)) - 0.5 * r(704),

}

def continuum_removed(wl, s, lo, hi):

"""Band depth and area of an absorption feature after convex-hull continuum removal."""

m = (wl >= lo) & (wl <= hi)

x, y = wl[m], s[m]

if x.size < 5:

return np.nan, np.nan

hull = [0]

for k in range(1, len(x)):                      # upper convex hull (monotone chain)

while len(hull) >= 2:

a, b = hull[-2], hull[-1]

if (y[b] - y[a]) * (x[k] - x[a]) <= (y[k] - y[a]) * (x[b] - x[a]):

hull.pop()

else:

break

hull.append(k)

cont = np.interp(x, x[hull], y[hull])

cr = y / cont

return float(1 - cr.min()), float(np.trapezoid(1 - cr, x))

def hs_features(arr, inside, veg, wl):

spec = np.nanmean(arr[:, veg], axis=1) if veg.any() else np.full(len(wl), np.nan)

f = {f"HS_R{w:.0f}": v for w, v in zip(wl, spec)}

r = lambda nm: spec[band_index(wl, nm, tol=10)] if band_index(wl, nm, tol=10) is not None else np.nan

for k, fn in HS_VIS.items():

f[f"HS_{k}"] = float(fn(r))

d1 = np.gradient(spec, wl)

for edge, (lo, hi) in {"b": (490, 530), "y": (550, 582), "r": (680, 760)}.items():

m = (wl >= lo) & (wl <= hi)

if m.sum() >= 2:

k = np.argmax(d1[m])

f[f"HS_D{edge}"] = d1[m][k]; f[f"HS_lambda_{edge}"] = wl[m][k]

f[f"HS_SD{edge}"] = np.trapezoid(d1[m], wl[m])

if {"HS_SDr", "HS_SDb", "HS_SDy"} <= f.keys():

f["HS_SDr_SDb"] = safe_div(f["HS_SDr"], f["HS_SDb"])

f["HS_SDr_SDy"] = safe_div(f["HS_SDr"], f["HS_SDy"])

f["HS_REP"] = 700 + 40 * safe_div((r(670) + r(780)) / 2 - r(700), r(740) - r(700))

for name, (lo, hi) in {"670": (540, 760), "970": (920, 1080), "1200": (1100, 1280)}.items():

f[f"HS_CR{name}_depth"], f[f"HS_CR{name}_area"] = continuum_removed(wl, spec, lo, hi)

return f

# ------------------------------------------------------------------ RGB

def rgb_features(arr, inside, veg):

R, G, B = arr[0], arr[1], arr[2]

s = R + G + B + 1e-9

r, g, b = R / s, G / s, B / s

idx = {"ExG": 2 * g - r - b, "ExR": 1.4 * r - g, "VARI": safe_div(G - R, G + R - B),

"GLI": safe_div(2 * G - R - B, 2 * G + R + B), "NGRDI": safe_div(G - R, G + R)}

f = {f"RGB_{k}": np.nanmean(v[veg]) for k, v in idx.items()}

for i, band in enumerate(C.RGB_BANDS):

f.update(glcm_features(arr[i], veg, prefix=f"RGB_{band}_"))

return f

# ------------------------------------------------------------------ TIR

def tir_features(arr, inside, veg, air_temp, ct_min, ct_max):

t = arr[0]

ct = np.nanmean(t[veg]) if veg.any() else np.nan

f = {"TIR_CT": ct, "TIR_CTD": ct - air_temp, "TIR_CTR": ct / air_temp,

"TIR_NRCT": (ct - ct_min) / (ct_max - ct_min)}

f.update(stats(t[veg], "TIR_T"))

f.update(glcm_features(t, veg, prefix="TIR_"))

return f

# ------------------------------------------------------------------ driver

MIN_VEG_PIXELS = 20

def extract(sensor):

low_cover = []

plots = gpd.read_file(C.PLOTS_FILE)

plots["geometry"] = plots.geometry.buffer(-C.PLOT_INSET_M, join_style="mitre")

weather = pd.read_csv(C.WEATHER_CSV, dtype={"date": str}).set_index("date") if C.WEATHER_CSV.exists() else None

wl = np.loadtxt(C.PRE / "HS" / "wavelengths.txt") if sensor == "HS" else None

rows = []

for date in C.DATES:

img_path = C.PRE / sensor / f"{sensor}_{date}.tif"

if not img_path.exists():

continue

ab_path = C.UNMIX / f"{sensor}_{date}_abundance.tif"

ab_names = pd.read_csv(C.UNMIX / f"{sensor}_endmembers.csv", nrows=0).columns.tolist() if ab_path.exists() else []

with rasterio.open(img_path) as src, rasterio.open(C.MASK / sensor / f"{sensor}_{date}_veg.tif") as msk:

if sensor == "TIR":     # field-wide canopy temperature range on this date (robust min/max)

t_all = src.read(1); vm = msk.read(1) == 1

ct_min, ct_max = np.nanpercentile(t_all[vm], [0.5, 99.5])

air = weather.loc[date, "air_temp_C"] if weather is not None else np.nan

ab_src = rasterio.open(ab_path) if ab_path.exists() else None

for _, p in plots.iterrows():

arr, inside, veg = plot_pixels(src, p.geometry, msk)

row = {"plot_id": int(p.plot_id), "date": date,

f"{sensor}_cover": veg.sum() / max(inside.sum(), 1)}

if veg.sum() < MIN_VEG_PIXELS:

# severe disease can push the whole canopy below the NDVI threshold;

# use all plot pixels rather than lose the most diseased plots

veg = inside.copy()

low_cover.append((sensor, date, int(p.plot_id)))

if sensor == "MS":

row.update(ms_features(arr, inside, veg))

elif sensor == "HS":

row.update(hs_features(arr, inside, veg, wl))

elif sensor == "RGB":

row.update(rgb_features(arr, inside, veg))

elif sensor == "TIR":

row.update(tir_features(arr, inside, veg, air, ct_min, ct_max))

if ab_src is not None:

a, ins, _ = plot_pixels(ab_src, p.geometry)

means = {n: np.nanmean(a[k][ins]) for k, n in enumerate(ab_names)}

row.update({f"{sensor}_ab_{n}": v for n, v in means.items()})

if "healthy" in means and "diseased" in means:

row[f"{sensor}_ab_diseased_share"] = means["diseased"] / (means["diseased"] + means["healthy"] + 1e-9)

rows.append(row)

if ab_src is not None:

ab_src.close()

print(f"{sensor} {date}: features for {len(plots)} plots")

df = pd.DataFrame(rows)

if low_cover:

print(f"  {sensor}: {len(low_cover)} plot-dates had < {MIN_VEG_PIXELS} vegetation pixels; "

f"all plot pixels used instead: {low_cover}")

C.FEAT.mkdir(parents=True, exist_ok=True)

df.to_csv(C.FEAT / f"features_{sensor}.csv", index=False)

return df

if __name__ == "__main__":

for s in C.SENSORS:

extract(s)

# 9. Step 6: Phase 1, disease-specific feature selection (s06_dsf.py)

Run separately for each sensor. Collinear features are kept in this phase (all-relevant problem); redundancy is removed in Step 7.

| Sub-step | Method | Rule (from config.py) |
| --- | --- | --- |
| 6.1 | Quality control: > 5 % missing, near-zero variance, modified z-score outliers (flagged, not removed); median imputation within date | — |
| 6.2 | Kruskal–Wallis across severity classes on each date, Benjamini–Hochberg FDR; Welch t-test healthy vs diseased (reported); ε² effect size | q < 0.05 on ≥ 2 of 4 dates |
| 6.3 | Jeffries–Matusita distance, healthy vs each severity class; HS separability scalogram | JM > 1.94 (HS bands), JM ≥ 1.0 (derived) |
| 6.4 | ReliefF (k = 10, multi-class) | Weight > 0 and in top half |
| 6.5 | RF permutation importance (plot-grouped CV); Boruta (shadow features, Bonferroni binomial test) | Top half; confirmed |
| 6.6 | Consensus: 6.2 and 6.3 passed, ≥ 2 of 3 votes from 6.4–6.5; stability over 50 random half-samples of plots | Stability ≥ 0.6 |
| 6.7 | Spearman correlation with DI; hierarchical clustering at ¦ρ¦ = 0.7 (interpretation only) | — |

select_dsf() is written so that it can be re-run inside every outer cross-validation fold (Step 7), which is what keeps the accuracy estimate free of selection bias. In that use fast=True skips the stability loop and uses fewer Boruta iterations.

Outputs (full data, for the thesis). 06_dsf/<sensor>_feature_table.csv (every statistic, vote, stability, cluster and the DSF flag), <sensor>_qc_removed.csv, and HS_JM_scalogram.png.

File: rustwatch_pipeline/s06_dsf.py

"""Step 6 - Phase 1: Disease-Specific Feature (DSF) selection, run separately for each sensor.

6.1 quality control (missing values, near-zero variance, outlier flags)

6.2 Kruskal-Wallis across severity classes on each date + Benjamini-Hochberg FDR

(+ Welch t-test healthy vs diseased, reported)

6.3 Jeffries-Matusita distance healthy vs each severity class; HS separability scalogram

6.4 ReliefF (multivariate filter)

6.5 Random-forest permutation importance (grouped CV) and Boruta (all-relevant)

6.6 consensus rule + stability over random half-samples of plots -> candidate DSFs

6.7 Spearman correlation with DI and correlation clustering (interpretation only)

Collinear features are NOT removed in this phase.

"""

import warnings

import numpy as np

import pandas as pd

from scipy.stats import kruskal, ttest_ind, spearmanr, binom

from scipy.spatial.distance import cdist, squareform

from scipy.cluster.hierarchy import linkage, fcluster

from statsmodels.stats.multitest import multipletests

from sklearn.ensemble import RandomForestClassifier

from sklearn.inspection import permutation_importance

from sklearn.model_selection import GroupKFold

import matplotlib

matplotlib.use("Agg")

import matplotlib.pyplot as plt

import config as C

from s01_layout import build_layout_table, load_ground_truth

warnings.filterwarnings("ignore", category=RuntimeWarning)

TARGET = "severity_class"

ID_COLS = ["plot_id", "date"]

# ------------------------------------------------------------------ data

def load_table(sensors):

"""Ground truth merged with the feature tables of the given sensors (one row per plot x date)."""

df = load_ground_truth(build_layout_table())

cols = []

for s in sensors:

f = pd.read_csv(C.FEAT / f"features_{s}.csv", dtype={"date": str})

cols += [c for c in f.columns if c not in ID_COLS]

df = df.merge(f, on=ID_COLS, how="inner")

return df.reset_index(drop=True), cols

# ------------------------------------------------------------------ 6.1 quality control

def quality_control(df, cols, max_missing=0.05):

keep, report = [], []

for c in cols:

x = df[c]

if x.isna().mean() > max_missing:

report.append((c, "missing")); continue

vc = x.round(10).value_counts()

freq_ratio = vc.iloc[0] / vc.iloc[1] if len(vc) > 1 else np.inf

if freq_ratio > 19 and 100 * len(vc) / len(x) < 10:

report.append((c, "near-zero variance")); continue

keep.append(c)

df = df.copy()

df[keep] = df.groupby("date")[keep].transform(lambda s: s.fillna(s.median()))

med = df.groupby("date")[keep].transform("median")

mad = df.groupby("date")[keep].transform(lambda s: (s - s.median()).abs().median())

mz = 0.6745 * (df[keep] - med) / mad.replace(0, np.nan)

n_out = int((mz.abs() > 3.5).any(axis=1).sum())

return df, keep, pd.DataFrame(report, columns=["feature", "reason"]), n_out

# ------------------------------------------------------------------ 6.2 statistics + FDR

def kruskal_p(df, c, target=TARGET):

groups = [g[c].values for _, g in df.groupby(target) if len(g) >= 2]

if len(groups) < 2:

return np.nan, np.nan

try:

h, p = kruskal(*groups)

except ValueError:            # all values identical

return 0.0, 1.0

return h, p

def stat_screen(df, cols, target=TARGET):

res = pd.DataFrame(index=cols)

sig_counts = pd.Series(0, index=cols)

for date, g in df.groupby("date"):

hp = np.array([kruskal_p(g, c, target) for c in cols])

p = np.nan_to_num(hp[:, 1], nan=1.0)

q = multipletests(p, method="fdr_bh")[1]

res[f"q_{date}"] = q

sig_counts += (q < C.FDR_Q).astype(int)

hd = g[target] > 0

res[f"t_p_{date}"] = [ttest_ind(g.loc[hd, c], g.loc[~hd, c], equal_var=False).pvalue

if hd.sum() > 1 and (~hd).sum() > 1 else np.nan for c in cols]

hp_all = np.array([kruskal_p(df, c, target) for c in cols])

res["H_pooled"] = hp_all[:, 0]

res["q_pooled"] = multipletests(np.nan_to_num(hp_all[:, 1], nan=1.0), method="fdr_bh")[1]

res["epsilon2"] = res["H_pooled"] / (len(df) - 1)

res["n_dates_sig"] = sig_counts

return res

# ------------------------------------------------------------------ 6.3 Jeffries-Matusita

def jm_1d(x1, x2):

m1, m2 = np.mean(x1), np.mean(x2)

v1, v2 = np.var(x1, ddof=1) + 1e-12, np.var(x2, ddof=1) + 1e-12

b = (m1 - m2) ** 2 / (4 * (v1 + v2)) + 0.5 * np.log((v1 + v2) / (2 * np.sqrt(v1 * v2)))

return 2 * (1 - np.exp(-b))

def jm_table(df, cols, target=TARGET):

healthy = df[df[target] == 0]

out = pd.DataFrame(index=cols)

for k in sorted(df[target].unique()):

if k == 0:

continue

d = df[df[target] == k]

if len(d) >= 3 and len(healthy) >= 3:

out[f"JM_0v{k}"] = [jm_1d(healthy[c].values, d[c].values) for c in cols]

out["JM_max"] = out.max(axis=1) if out.shape[1] else np.nan

return out

def is_hs_band(c):

return c.startswith("HS_R") and c[4:].replace(".", "").isdigit()

def plot_scalogram(table, path):

bands = [c for c in table.index if is_hs_band(c)]

if not bands:

return

wl = np.array([float(c[4:]) for c in bands])

o = np.argsort(wl)

fig, ax = plt.subplots(figsize=(8, 3.5))

for col in [c for c in table.columns if c.startswith("JM_0v")]:

ax.plot(wl[o], table.loc[bands, col].values[o], lw=1, label=col.replace("JM_0v", "healthy vs class "))

ax.axhline(C.JM_MIN_HS_BAND, color="k", ls="--", lw=1, label=f"JM = {C.JM_MIN_HS_BAND}")

ax.set_xlabel("Wavelength (nm)"); ax.set_ylabel("JM distance"); ax.set_ylim(0, 2.05)

ax.legend(fontsize=7, ncol=3); fig.tight_layout(); fig.savefig(path, dpi=150); plt.close(fig)

# ------------------------------------------------------------------ 6.4 ReliefF

def relieff(X, y, k=C.RELIEF_K):

X = np.asarray(X, float)

rng_ = X.max(0) - X.min(0)

X = (X - X.min(0)) / np.where(rng_ == 0, 1, rng_)

n, p = X.shape

classes, counts = np.unique(y, return_counts=True)

prior = dict(zip(classes, counts / n))

D = cdist(X, X, "cityblock")

np.fill_diagonal(D, np.inf)

W = np.zeros(p)

for i in range(n):

same = np.where(y == y[i])[0]

same = same[same != i]

if len(same):

h = same[np.argsort(D[i, same])[:k]]

W -= np.abs(X[h] - X[i]).sum(0) / (n * len(h))

for c in classes:

if c == y[i]:

continue

other = np.where(y == c)[0]

m = other[np.argsort(D[i, other])[:k]]

W += prior[c] / (1 - prior[y[i]]) * np.abs(X[m] - X[i]).sum(0) / (n * len(m))

return W

# ------------------------------------------------------------------ 6.5 RF importance + Boruta

def rf_permutation_importance(X, y, groups, n_trees=300, seed=C.RANDOM_STATE):

imps = []

for tr, te in GroupKFold(n_splits=C.INNER_FOLDS).split(X, y, groups):

rf = RandomForestClassifier(n_trees, class_weight="balanced", random_state=seed, n_jobs=-1).fit(X[tr], y[tr])

r = permutation_importance(rf, X[te], y[te], n_repeats=5, random_state=seed, scoring="f1_macro", n_jobs=-1)

imps.append(r.importances_mean)

return np.mean(imps, 0)

def boruta(X, y, n_iter=60, n_trees=200, alpha=0.01, seed=C.RANDOM_STATE):

"""Shadow-feature test (Kursa & Rudnicki, 2010). Returns 'confirmed' / 'tentative' / 'rejected'."""

rng = np.random.default_rng(seed)

n, p = X.shape

hits = np.zeros(p)

for it in range(n_iter):

shadow = rng.permuted(X, axis=0)

rf = RandomForestClassifier(n_trees, class_weight="balanced", random_state=seed + it, n_jobs=-1)

rf.fit(np.hstack([X, shadow]), y)

imp = rf.feature_importances_

hits += imp[:p] > imp[p:].max()

a = alpha / p                                           # Bonferroni

confirmed = binom.sf(hits - 1, n_iter, 0.5) < a

rejected = binom.cdf(hits, n_iter, 0.5) < a

return np.where(confirmed, "confirmed", np.where(rejected, "rejected", "tentative")), hits / n_iter

# ------------------------------------------------------------------ 6.6 consensus + stability

def stability(df, cols, n_sub=C.BOOTSTRAP_N, seed=C.RANDOM_STATE):

"""Fraction of random half-samples of plots in which a feature passes FDR and is in the ReliefF top half."""

rng = np.random.default_rng(seed)

plots = df.plot_id.unique()

freq = np.zeros(len(cols))

for _ in range(n_sub):

sub = df[df.plot_id.isin(rng.choice(plots, len(plots) // 2, replace=False))]

p = np.array([kruskal_p(sub, c)[1] for c in cols])

q = multipletests(np.nan_to_num(p, nan=1.0), method="fdr_bh")[1]

w = relieff(sub[cols].values, sub[TARGET].values)

freq += (q < C.FDR_Q) & (w > 0) & (w >= np.median(w))

return freq / n_sub

def select_dsf(df, cols, fast=False, out_prefix=None):

"""Phase 1 for one sensor. Returns (DSF list, full table, consensus rank Series)."""

df, cols, qc_report, n_out = quality_control(df, cols)

X, y, g = df[cols].values, df[TARGET].values, df.plot_id.values

t = stat_screen(df, cols)

t = t.join(jm_table(df, cols))

t["relief"] = relieff(X, y)

t["rf_perm"] = rf_permutation_importance(X, y, g, n_trees=100 if fast else 300)

t["boruta"], t["boruta_hits"] = boruta(X, y, n_iter=20 if fast else 60, n_trees=100 if fast else 200)

t["pass_stat"] = t["n_dates_sig"] >= C.MIN_DATES_SIGNIFICANT

jm_thr = np.where([is_hs_band(c) for c in cols], C.JM_MIN_HS_BAND, C.JM_MIN_DERIVED)

t["pass_sep"] = t["JM_max"].fillna(0).values >= jm_thr

t["vote_relief"] = (t["relief"] > 0) & (t["relief"] >= t["relief"].median())

t["vote_rf"] = t["rf_perm"] >= t["rf_perm"].median()

t["vote_boruta"] = t["boruta"] == "confirmed"

t["votes"] = t[["vote_relief", "vote_rf", "vote_boruta"]].sum(1)

candidate = t["pass_stat"] & t["pass_sep"] & (t["votes"] >= 2)

if not fast:

t["stability"] = 0.0

if candidate.any():

t.loc[candidate, "stability"] = stability(df, list(t.index[candidate]))

candidate &= t["stability"] >= C.STABILITY_MIN

t["DSF"] = candidate

ranks = pd.concat([t["q_pooled"].rank(), t["JM_max"].rank(ascending=False),

t["relief"].rank(ascending=False), t["rf_perm"].rank(ascending=False)], axis=1)

t["consensus_rank"] = ranks.mean(1).rank()

dsf = list(t.index[t["DSF"]])

if len(dsf) < 3:          # never leave a sensor empty: fall back to the best-ranked features

dsf = list(t.sort_values("consensus_rank").index[:5])

t["DSF_fallback"] = t.index.isin(dsf)

if out_prefix is not None:

C.DSF.mkdir(parents=True, exist_ok=True)

t["rho_DI"] = [spearmanr(df[c], df["DI"])[0] for c in cols]

if len(dsf) > 2:

rho = df[dsf].corr("spearman").abs().fillna(0).values

dist = squareform(1 - rho, checks=False)

t.loc[dsf, "cluster"] = fcluster(linkage(dist, "average"), t=0.3, criterion="distance")

t.sort_values("consensus_rank").to_csv(C.DSF / f"{out_prefix}_feature_table.csv")

qc_report.to_csv(C.DSF / f"{out_prefix}_qc_removed.csv", index=False)

plot_scalogram(t, C.DSF / f"{out_prefix}_JM_scalogram.png")

print(f"{out_prefix}: {len(cols)} features after QC, {n_out} plot-dates flagged as outliers, "

f"{len(dsf)} DSFs")

return dsf, t, t["consensus_rank"]

if __name__ == "__main__":

for s in C.SENSORS:

d, cols = load_table([s])

dsf, _, _ = select_dsf(d, cols, out_prefix=s)

print(s, "DSFs:", dsf[:15], "..." if len(dsf) > 15 else "")

# 10. Step 7: Phases 2–3, redundancy removal, SFFS and nested CV (s07_models.py)

- Relevance-guided VIF: among features with VIF ≥ 10 the one with the worst Phase 1 consensus rank is dropped, then VIF is recomputed. VIF is computed as the diagonal of the inverse correlation matrix.

- SPA (Successive Projections Algorithm) for raw hyperspectral bands that pass Phase 1, because VIF fails when bands ≫ plots; the chain length is chosen by grouped-CV macro-F1 of LDA (SPA-LDA).

- SFFS with mlxtend (floating=True), scoring macro-F1, inner CV grouped by plot; the subset size is chosen by the one-standard-error rule. Each classifier gets its own OFC.

- Tuning of the classifier on its OFC by grid search (inner grouped CV).

- Nested CV: StratifiedGroupKFold (4 outer folds, 12 plots each, repeated 3 times). In every outer fold, Phase 1 → SPA → VIF → SFFS → tuning run on the training plots only; the held-out plots are predicted once.

- Metrics: overall accuracy, macro-F1 and Cohen’s kappa per repeat (mean ± SD); per-class precision, recall and specificity.

Classifiers: Random Forest (balanced class weights), SVM with RBF kernel, and KNN, each in a pipeline with median imputation and standardisation (the scaler is fitted on training folds only). Other scikit-learn classifiers can be added to classifiers() with their own grid; they automatically get their own SFFS run.

File: rustwatch_pipeline/s07_models.py

"""Step 7 - Phases 2 and 3: redundancy removal, SFFS, classifiers and nested grouped CV.

7.1 relevance-guided VIF (< 10)          (per sensor in Phase 2, pooled in Phase 3)

7.2 SPA for raw hyperspectral bands      (bands are p >> n, so VIF cannot be used)

7.3 SFFS per classifier (mlxtend, floating=True), inner CV grouped by plot,

subset size by the one-standard-error rule -> OFC

7.4 hyperparameter tuning on the OFC (GridSearchCV, inner grouped CV)

7.5 nested, plot-grouped, repeated outer CV; Phase 1 is re-run inside every outer

training fold so no label information leaks into the test fold

7.6 metrics: OA, macro-F1, kappa, per-class precision / recall / specificity

"""

import os

import warnings

import numpy as np

import pandas as pd

from sklearn.base import clone

from sklearn.pipeline import Pipeline

from sklearn.impute import SimpleImputer

from sklearn.preprocessing import StandardScaler

from sklearn.ensemble import RandomForestClassifier

from sklearn.svm import SVC

from sklearn.neighbors import KNeighborsClassifier

from sklearn.discriminant_analysis import LinearDiscriminantAnalysis

from sklearn.model_selection import GroupKFold, StratifiedGroupKFold, GridSearchCV, cross_val_score

from sklearn.metrics import accuracy_score, f1_score, cohen_kappa_score, confusion_matrix

from mlxtend.feature_selection import SequentialFeatureSelector as SFS

import config as C

from s06_dsf import select_dsf, is_hs_band, TARGET

warnings.filterwarnings("ignore")

FAST = os.environ.get("RW_FAST") == "1"     # quick settings for testing the code

def pipe(clf):

return Pipeline([("imp", SimpleImputer(strategy="median")), ("sc", StandardScaler()), ("clf", clf)])

def classifiers():

rs = C.RANDOM_STATE

n_trees = 60 if FAST else 300

return {

"RF": (pipe(RandomForestClassifier(n_trees, class_weight="balanced", random_state=rs, n_jobs=-1)),

{"clf__max_features": ["sqrt", 0.5], "clf__min_samples_leaf": [1, 3]}),

"SVM": (pipe(SVC(kernel="rbf", class_weight="balanced", random_state=rs)),

{"clf__C": 2.0 ** np.arange(-3, 12, 2), "clf__gamma": 2.0 ** np.arange(-11, 2, 2)}),

"KNN": (pipe(KNeighborsClassifier()),

{"clf__n_neighbors": [3, 5, 7, 9, 11, 15], "clf__weights": ["uniform", "distance"]}),

}

def inner_cv(groups, n=None):

return GroupKFold(n_splits=n or C.INNER_FOLDS)

# ------------------------------------------------------------------ 7.1 VIF

def vif_values(X):

R = np.corrcoef(np.asarray(X, float), rowvar=False)

return np.diag(np.linalg.pinv(R))              # VIF_j = (R^-1)_jj

def vif_filter(X, rank, thresh=C.VIF_MAX):

"""Iteratively drop, among features with VIF >= thresh, the one with the WORST Phase 1 rank."""

feats = list(X.columns)

removed = []

while len(feats) > 1:

v = vif_values(X[feats].fillna(X[feats].median()))

if np.nanmax(v) < thresh:

break

high = [f for f, vi in zip(feats, v) if vi >= thresh]

drop = max(high, key=lambda f: rank.get(f, np.inf))

feats.remove(drop)

removed.append(drop)

return feats, removed

# ------------------------------------------------------------------ 7.2 SPA

def spa_chains(X, max_vars):

"""Successive Projections Algorithm (Araujo et al., 2001): one chain per starting band."""

Xc = X - X.mean(0)

p = Xc.shape[1]

starts = range(p) if p <= 60 else np.linspace(0, p - 1, 60).astype(int)

chains = []

for k0 in starts:

sel, Xp = [int(k0)], Xc.copy()

for _ in range(min(max_vars, p) - 1):

xk = Xp[:, sel[-1]].copy()

Xp = Xp - np.outer(xk, xk @ Xp) / (xk @ xk + 1e-12)

norms = np.linalg.norm(Xp, axis=0)

norms[sel] = -1

sel.append(int(np.argmax(norms)))

chains.append(sel)

return chains

def spa_select(Xdf, y, groups, max_vars=10):

"""Pick the chain prefix with the best grouped-CV accuracy of LDA (SPA-LDA, Pontes et al., 2005)."""

X = Xdf.fillna(Xdf.median()).values

cv = list(inner_cv(groups).split(X, y, groups))

best, best_set = -np.inf, None

for chain in spa_chains(X, max_vars):

for m in range(2, len(chain) + 1):

s = cross_val_score(LinearDiscriminantAnalysis(), X[:, chain[:m]], y, cv=cv, scoring="f1_macro").mean()

if s > best:

best, best_set = s, chain[:m]

return [Xdf.columns[i] for i in best_set]

# ------------------------------------------------------------------ 7.3 SFFS

def sffs(estimator, X, y, groups, max_k=C.SFFS_MAX_FEATURES):

cv = list(inner_cv(groups).split(X, y, groups))

k_hi = min(max_k if not FAST else 6, X.shape[1])

sfs = SFS(clone(estimator), k_features=(1, k_hi), forward=True, floating=True,

scoring="f1_macro", cv=cv, n_jobs=-1)

sfs.fit(X, y)

res = sfs.subsets_

best_k = max(res, key=lambda k: res[k]["avg_score"])

se = np.std(res[best_k]["cv_scores"], ddof=1) / np.sqrt(len(cv))

k_sel = min(k for k in res if res[k]["avg_score"] >= res[best_k]["avg_score"] - se)   # one-SE rule

return list(res[k_sel]["feature_names"]), pd.DataFrame(

[(k, v["avg_score"], ";".join(v["feature_names"])) for k, v in res.items()], columns=["k", "macro_F1", "features"])

# ------------------------------------------------------------------ 7.4 per-fold model building

def sensor_of(col):

return col.split("_")[0]

def dsf_for_sensor(train, sensor_cols, sensor, fast=True):

dsf, _, rank = select_dsf(train, sensor_cols, fast=fast)

if sensor == "HS":

bands = [f for f in dsf if is_hs_band(f)]

if len(bands) > 10:

bands = spa_select(train[bands], train[TARGET].values, train.plot_id.values)

dsf = bands + [f for f in dsf if not is_hs_band(f)]

return dsf, rank

def build_model(train, dsf_by_sensor, rank_by_sensor, sensors, clf_name):

feats = [f for s in sensors for f in dsf_by_sensor[s]]

rank = pd.concat([rank_by_sensor[s] for s in sensors])

feats, _ = vif_filter(train[feats], rank)

est, grid = classifiers()[clf_name]

y, g = train[TARGET].values, train.plot_id.values

ofc, _ = sffs(est, train[feats], y, g)

gs = GridSearchCV(clone(est), grid, scoring="f1_macro", cv=list(inner_cv(g).split(train, y, g)), n_jobs=-1)

gs.fit(train[ofc], y)

return dict(model=gs.best_estimator_, ofc=ofc, after_vif=feats, params=gs.best_params_)

# ------------------------------------------------------------------ 7.5 nested CV

def run_nested(df, feature_cols, configs, clf_names=("RF", "SVM", "KNN"), n_repeats=None, outer_folds=None):

"""configs: dict name -> list of sensors, e.g. {"MS": ["MS"], "F2_MS+TIR": ["MS", "TIR"]}.

Phase 1 runs once per sensor per outer fold and is shared by all configurations."""

y, g = df[TARGET].values, df.plot_id.values

preds, picks = [], []

sensors_needed = sorted({s for v in configs.values() for s in v})

for rep in range(n_repeats or (1 if FAST else C.N_REPEATS)):

outer = StratifiedGroupKFold(n_splits=outer_folds or C.OUTER_FOLDS, shuffle=True, random_state=C.RANDOM_STATE + rep)

for fold, (tr, te) in enumerate(outer.split(df, y, g)):

train, test = df.iloc[tr], df.iloc[te]

dsf, rank = {}, {}

for s in sensors_needed:

dsf[s], rank[s] = dsf_for_sensor(train, [c for c in feature_cols if sensor_of(c) == s], s)

for name, sensors in configs.items():

for clf in clf_names:

m = build_model(train, dsf, rank, sensors, clf)

yp = m["model"].predict(test[m["ofc"]])

preds.append(pd.DataFrame(dict(config=name, clf=clf, repeat=rep, fold=fold, row=test.index,

plot_id=test.plot_id.values, date=test.date.values,

y_true=test[TARGET].values, y_pred=yp)))

picks.append(dict(config=name, clf=clf, repeat=rep, fold=fold, n_after_vif=len(m["after_vif"]),

ofc=";".join(m["ofc"]), params=str(m["params"])))

print(f"rep {rep} fold {fold} {name:<14} {clf:<4} OFC={len(m['ofc'])} "

f"F1={f1_score(test[TARGET], yp, average='macro', zero_division=0):.2f}")

return pd.concat(preds, ignore_index=True), pd.DataFrame(picks)

# ------------------------------------------------------------------ 7.6 metrics

def specificity_per_class(cm):

out = []

for k in range(cm.shape[0]):

tn = cm.sum() - cm[k].sum() - cm[:, k].sum() + cm[k, k]

fp = cm[:, k].sum() - cm[k, k]

out.append(tn / (tn + fp) if tn + fp else np.nan)

return out

def summarize(preds):

rows = []

for (cfg, clf), d in preds.groupby(["config", "clf"]):

per_rep = d.groupby("repeat").apply(lambda r: pd.Series(dict(

OA=accuracy_score(r.y_true, r.y_pred),

macro_F1=f1_score(r.y_true, r.y_pred, average="macro", zero_division=0),

kappa=cohen_kappa_score(r.y_true, r.y_pred))), include_groups=False)

rows.append(dict(config=cfg, clf=clf, **{f"{m}_mean": per_rep[m].mean() for m in per_rep},

**{f"{m}_sd": per_rep[m].std(ddof=1) for m in per_rep}))

return pd.DataFrame(rows).sort_values("macro_F1_mean", ascending=False)

def class_report(preds, cfg, clf):

d = preds[(preds.config == cfg) & (preds.clf == clf)]

labels = sorted(set(d.y_true) | set(d.y_pred))

cm = confusion_matrix(d.y_true, d.y_pred, labels=labels)

rec = np.diag(cm) / cm.sum(1).clip(min=1)

prec = np.diag(cm) / cm.sum(0).clip(min=1)

return cm, pd.DataFrame(dict(cls=labels, precision=prec, recall=rec, specificity=specificity_per_class(cm)))

# 11. Step 8: Fusion, final model, SHAP, maps and DI regression (s08_fusion_maps.py)

| Configuration | Sensors | Phase |
| --- | --- | --- |
| MS, HS, RGB, TIR | one sensor each | Phase 2 (single sensor) |
| F1_RGB+TIR | RGB + thermal | Phase 3 |
| F2_MS+TIR | Multispectral + thermal | Phase 3 |
| F3_HS+TIR | Hyperspectral + thermal | Phase 3 |
| F4_ALL | RGB + MS + HS + thermal | Phase 3 |

- All 8 configurations × 3 classifiers are evaluated with the same outer folds, so results are paired. Per-sensor DSFs are computed once per fold and shared by single-sensor and fusion configurations.

- Fusion gain = macro-F1 (and OA) of each fusion scenario minus that of the best single sensor for the same classifier, with an exact McNemar test on the paired predictions.

- Final model: Phase 1 with stability, VIF, SFFS and tuning on all 192 observations for the best configuration. Robust cross-classifier features: features chosen in at least half of the outer folds by at least two classifiers.

- SHAP: TreeSHAP for Random Forest, Kernel SHAP for SVM and KNN; mean |SHAP| per feature and its share per sensor.

- Maps: predicted severity class and the three spray zones (classes 0–1 Green: skip; 2–3 Amber: spray at label dose; 4–5 Red: too late) for each date, as PNG and GeoPackage. Agree the class-to-zone rule with plant pathologists before using it.

- DI regression: Random Forest regression of the continuous DI on the DSFs of the best configuration, plot-grouped CV; R², RMSE, RE (%).

The maps are made with the final model applied to the same plots it was trained on, so they illustrate the output but are not an accuracy estimate. Accuracy comes only from the nested CV and from the independent fields (IIRMR Bharatpur, farmers’ fields): run Steps 2–5 on those images and call fm["model"].predict() on their features.

File: rustwatch_pipeline/s08_fusion_maps.py

"""Step 8 - Single-sensor vs multi-sensor comparison, final model, SHAP, maps, regression.

8.1 nested CV for the 4 single sensors (Phase 2) and the 4 fusion scenarios (Phase 3)

8.2 fusion gain = best fused macro-F1 - best single-sensor macro-F1; McNemar test

8.3 final model on all data for the best configuration; robust cross-classifier features

8.4 SHAP explanation and sensor contributions

8.5 severity map and three-zone spray map for every flight date

8.6 regression of the continuous Disease Index (R2, RMSE, RE) with grouped CV

"""

import numpy as np

import pandas as pd

import geopandas as gpd

import shap

from collections import Counter

from statsmodels.stats.contingency_tables import mcnemar

from sklearn.ensemble import RandomForestRegressor

from sklearn.calibration import CalibratedClassifierCV

from sklearn.base import clone

from sklearn.model_selection import GroupKFold

from sklearn.metrics import r2_score, mean_squared_error

import matplotlib

matplotlib.use("Agg")

import matplotlib.pyplot as plt

from matplotlib.patches import Patch

import config as C

from s06_dsf import load_table, TARGET

from s07_models import (run_nested, summarize, class_report, dsf_for_sensor, build_model, sensor_of, FAST)

SINGLE = {s: [s] for s in C.SENSORS}

FUSION = {"F1_RGB+TIR": ["RGB", "TIR"], "F2_MS+TIR": ["MS", "TIR"],

"F3_HS+TIR": ["HS", "TIR"], "F4_ALL": ["RGB", "MS", "HS", "TIR"]}

ZONE = {0: "Green", 1: "Green", 2: "Amber", 3: "Amber", 4: "Red", 5: "Red"}   # agree with pathologists

ZONE_COLOUR = {"Green": "#3E7B4F", "Amber": "#E0A526", "Red": "#A63A2A"}

CLASS_COLOUR = ["#2E7D32", "#9CCC65", "#FFEE58", "#FFA726", "#EF5350", "#8E0000"]

# ------------------------------------------------------------------ 8.2 fusion gain + McNemar

def mcnemar_test(preds, a, b, repeat=0):

"""a, b = (config, clf). Paired on the same rows of the same outer-CV repeat."""

pa = preds[(preds.config == a[0]) & (preds.clf == a[1]) & (preds.repeat == repeat)].set_index("row")

pb = preds[(preds.config == b[0]) & (preds.clf == b[1]) & (preds.repeat == repeat)].set_index("row")

ca, cb = (pa.y_true == pa.y_pred), (pb.y_true == pb.y_pred).reindex(pa.index)

table = [[(ca & cb).sum(), (ca & ~cb).sum()], [(~ca & cb).sum(), (~ca & ~cb).sum()]]

return mcnemar(table, exact=True).pvalue

def fusion_gain(summary, preds):

single = summary[summary.config.isin(SINGLE)]

fused = summary[summary.config.isin(FUSION)]

rows = []

for clf in summary.clf.unique():

bs = single[single.clf == clf].iloc[0]

for _, f in fused[fused.clf == clf].iterrows():

rows.append(dict(clf=clf, fusion=f.config, best_single=bs.config,

gain_macro_F1=f.macro_F1_mean - bs.macro_F1_mean, gain_OA=f.OA_mean - bs.OA_mean,

mcnemar_p=mcnemar_test(preds, (f.config, clf), (bs.config, clf))))

return pd.DataFrame(rows)

# ------------------------------------------------------------------ 8.3 final model

def final_model(df, cols, sensors, clf):

dsf, rank = {}, {}

for s in sensors:

dsf[s], rank[s] = dsf_for_sensor(df, [c for c in cols if sensor_of(c) == s], s, fast=False)

return build_model(df, dsf, rank, sensors, clf)

def robust_features(picks, config):

"""Features chosen in >= 2 of the 3 classifiers' OFCs, counted over all outer folds."""

d = picks[picks.config == config]

per_clf = {clf: Counter(f for o in g.ofc for f in o.split(";")) for clf, g in d.groupby("clf")}

n_folds = d.groupby("clf").size().max()

feats = set().union(*per_clf.values())

rows = [dict(feature=f, **{f"{clf}_freq": per_clf[clf][f] / n_folds for clf in per_clf}) for f in feats]

t = pd.DataFrame(rows)

t["n_classifiers"] = (t.filter(like="_freq") >= 0.5).sum(1)

return t.sort_values("n_classifiers", ascending=False)

# ------------------------------------------------------------------ 8.4 SHAP

def probability_function(clf, Xs, y):

"""Class probabilities for Kernel SHAP. SVC has no predict_proba unless calibrated."""

try:

clf.predict_proba(Xs[:1])

return clf.predict_proba

except AttributeError:

pass

n_min = int(pd.Series(y).value_counts().min())

if n_min >= 2:

cal = CalibratedClassifierCV(clone(clf), cv=min(3, n_min)).fit(Xs, y)

return cal.predict_proba

classes = np.unique(y)                              # last resort: explain the predicted label

return lambda X: (clf.predict(X)[:, None] == classes[None]).astype(float)

def shap_importance(model, X, y):

pre = model[:-1]

clf = model.named_steps["clf"]

Xs = pre.transform(X)

if clf.__class__.__name__ == "RandomForestClassifier":

sv = np.array(shap.TreeExplainer(clf).shap_values(Xs))

else:

bg = shap.kmeans(Xs, min(10, len(Xs)))

sv = np.array(shap.KernelExplainer(probability_function(clf, Xs, y), bg).shap_values(Xs, nsamples=100, silent=True))

n, p = X.shape

if sv.ndim == 2:

imp = np.abs(sv).mean(0)

elif sv.shape[0] == n and sv.shape[1] == p:      # (samples, features, classes)

imp = np.abs(sv).mean((0, 2))

else:                                           # (classes, samples, features)

imp = np.abs(sv).mean((0, 1))

s = pd.Series(imp, index=X.columns).sort_values(ascending=False)

contrib = s.groupby(lambda c: sensor_of(c)).sum()

return s, contrib / contrib.sum()

# ------------------------------------------------------------------ 8.5 maps

def make_maps(df, fm, tag):

plots = gpd.read_file(C.PLOTS_FILE)

C.MAPS.mkdir(parents=True, exist_ok=True)

out = []

for date, d in df.groupby("date"):

pred = pd.DataFrame(dict(plot_id=d.plot_id.values, pred_class=fm["model"].predict(d[fm["ofc"]]),

true_class=d[TARGET].values))

pred["zone"] = pred.pred_class.map(ZONE)

g = plots.merge(pred, on="plot_id")

g["date"] = date

out.append(g)

fig, axes = plt.subplots(1, 2, figsize=(13, 4.5))

g.plot(ax=axes[0], color=[CLASS_COLOUR[c] for c in g.pred_class], edgecolor="k", lw=0.5)

axes[0].legend(handles=[Patch(color=CLASS_COLOUR[k], label=f"{k} {n}") for k, n in enumerate(C.CLASS_NAMES)],

fontsize=7, loc="upper left", bbox_to_anchor=(1, 1))

axes[0].set_title(f"Predicted severity class, {date}")

g.plot(ax=axes[1], color=[ZONE_COLOUR[z] for z in g.zone], edgecolor="k", lw=0.5)

axes[1].legend(handles=[Patch(color=c, label=z) for z, c in ZONE_COLOUR.items()],

fontsize=8, loc="upper left", bbox_to_anchor=(1, 1))

axes[1].set_title("Spray decision: Green skip / Amber spray / Red too late")

for ax in axes:

for _, r in g.iterrows():

ax.annotate(str(r.plot_id), r.geometry.centroid.coords[0], ha="center", va="center", fontsize=6)

ax.set_axis_off()

fig.tight_layout(); fig.savefig(C.MAPS / f"{tag}_{date}_maps.png", dpi=150); plt.close(fig)

allg = pd.concat(out)

gpd.GeoDataFrame(allg, crs=plots.crs).to_file(C.MAPS / f"{tag}_predictions.gpkg", layer="predictions")

return allg

# ------------------------------------------------------------------ 8.6 regression of DI

def di_regression(df, cols, sensors, n_folds=None):

preds = np.full(len(df), np.nan)

g = df.plot_id.values

for tr, te in GroupKFold(n_splits=n_folds or C.OUTER_FOLDS).split(df, df.DI, g):

train = df.iloc[tr]

feats = []

for s in sensors:

feats += dsf_for_sensor(train, [c for c in cols if sensor_of(c) == s], s)[0]

rf = RandomForestRegressor(300, random_state=C.RANDOM_STATE, n_jobs=-1)

rf.fit(train[feats].fillna(train[feats].median()), train.DI)

preds[te] = rf.predict(df.iloc[te][feats].fillna(train[feats].median()))

rmse = mean_squared_error(df.DI, preds) ** 0.5

return dict(R2=float(r2_score(df.DI, preds)), RMSE=float(rmse), RE_pct=float(100 * rmse / df.DI.mean())), preds

# ------------------------------------------------------------------ driver

def main():

C.MODELS.mkdir(parents=True, exist_ok=True)

df, cols = load_table(C.SENSORS)

print(f"{len(df)} plot-date observations, {df.plot_id.nunique()} plots, {len(cols)} features")

configs = {**SINGLE, **FUSION}

preds, picks = run_nested(df, cols, configs)

preds.to_csv(C.MODELS / "outer_cv_predictions.csv", index=False)

picks.to_csv(C.MODELS / "selected_features_per_fold.csv", index=False)

summary = summarize(preds)

summary.to_csv(C.MODELS / "performance_summary.csv", index=False)

print(summary.round(3).to_string(index=False))

gain = fusion_gain(summary, preds)

gain.to_csv(C.MODELS / "fusion_gain.csv", index=False)

print(gain.round(3).to_string(index=False))

best = summary.iloc[0]

cm, rep = class_report(preds, best.config, best.clf)

pd.DataFrame(cm).to_csv(C.MODELS / "best_confusion_matrix.csv")

rep.to_csv(C.MODELS / "best_class_report.csv", index=False)

robust = robust_features(picks, best.config)

robust.to_csv(C.MODELS / f"robust_features_{best.config}.csv", index=False)

sensors = configs[best.config]

fm = final_model(df, cols, sensors, best.clf)

pd.Series(fm["ofc"]).to_csv(C.MODELS / "final_OFC.csv", index=False, header=["feature"])

print(f"Final model: {best.config} / {best.clf}; OFC = {fm['ofc']}; params = {fm['params']}")

imp, contrib = shap_importance(fm["model"], df[fm["ofc"]], df[TARGET].values)

imp.to_csv(C.MODELS / "shap_feature_importance.csv", header=["mean_abs_shap"])

contrib.to_csv(C.MODELS / "shap_sensor_contribution.csv", header=["share"])

print("Sensor contribution (SHAP):", contrib.round(2).to_dict())

make_maps(df, fm, f"{best.config}_{best.clf}")

reg, _ = di_regression(df, cols, sensors)

pd.Series(reg).to_csv(C.MODELS / "di_regression.csv", header=["value"])

print("DI regression:", {k: round(v, 3) for k, v in reg.items()})

if __name__ == "__main__":

main()

# 12. Running everything and testing with synthetic data

## 12.1 run_all.py

File: rustwatch_pipeline/run_all.py

"""Run the whole pipeline in order.   python run_all.py            (all steps)

python run_all.py 5 6        (only steps 5 and 6)"""

import sys

import config as C

import s01_layout, s02_preprocess, s03_masking, s04_endmembers, s05_features, s06_dsf, s08_fusion_maps

def step1():

layout = s01_layout.build_layout_table()

for msg in s01_layout.check_layout(layout):

print("LAYOUT CHECK:", msg)

layout.to_csv(C.LAYOUT_CSV, index=False)

if not C.PLOTS_FILE.exists():

s01_layout.build_plot_polygons(layout).drop(columns="geometry_inner").to_file(C.PLOTS_FILE, layer="plots")

STEPS = {

1: ("Field layout and plot polygons", step1),

2: ("Pre-processing and co-registration", s02_preprocess.preprocess_all),

3: ("Vegetation masking", lambda: [s03_masking.mask_date(d) for d in C.DATES]),

4: ("Endmember extraction and unmixing", lambda: s04_endmembers.run_unmixing("HS")),

5: ("Plot-wise feature extraction", lambda: [s05_features.extract(s) for s in C.SENSORS]),

6: ("Phase 1 DSF selection (full data, for reporting)",

lambda: [s06_dsf.select_dsf(*s06_dsf.load_table([s]), out_prefix=s) for s in C.SENSORS]),

7: ("Phases 2-3: nested CV, fusion, final model, SHAP, maps, regression", s08_fusion_maps.main),

}

if __name__ == "__main__":

todo = [int(a) for a in sys.argv[1:]] or list(STEPS)

for k in todo:

name, fn = STEPS[k]

print(f"\n===== Step {k}: {name} =====")

fn()

## 12.2 make_synthetic_data.py

Creates 16 orthomosaics, plot polygons, ground truth and weather with your exact structure (48 plots, 4 dates; control plots healthy; inoculated plots with DI depending on variety, inoculation age and date), with a deliberate 24 cm offset in the thermal images. Use it to test the code on your computer before the real data are ready: python make_synthetic_data.py then RW_FAST=1 python run_all.py. Delete the `project` folder before running with real data.

File: rustwatch_pipeline/make_synthetic_data.py

"""Create a small synthetic dataset with the same structure as the real one

(48 plots, 4 dates, MS / HS / RGB / TIR orthomosaics, ground truth, weather),

so that every step of the pipeline can be tested before real data are ready.

The spectra are simplified; results on these data say nothing about real accuracy."""

import numpy as np

import pandas as pd

import geopandas as gpd

from rasterio.features import rasterize

from rasterio.transform import from_origin

from scipy.ndimage import gaussian_filter

import config as C

from common import write_raster

from s01_layout import build_layout_table, build_plot_polygons

rng = np.random.default_rng(1)

FINE = 0.02                                   # m, base grid

MARGIN = 2.0

X0, Y0 = C.FIELD_ORIGIN_E - MARGIN, C.FIELD_ORIGIN_N + MARGIN

W, H = int((C.FIELD_WIDTH_M + 2 * MARGIN) / FINE), int((C.FIELD_LENGTH_M + 2 * MARGIN) / FINE)

FACTORS = {"RGB": 1, "MS": 2, "TIR": 4, "HS": 10}     # 2 cm, 4 cm, 8 cm, 20 cm

WL = np.arange(400, 2501, 15.0)

AIR = {d: t for d, t in zip(C.DATES, [20.0, 22.5, 19.0, 24.0])}

def spectra():

wl = WL

soil = 0.10 + 0.18 * (wl - 400) / 2100

def veg(s):

vis = 0.04 + 0.05 * np.exp(-((wl - 550) / 30) ** 2) - 0.015 * np.exp(-((wl - 670) / 25) ** 2)

vis += s * (0.05 * np.exp(-((wl - 600) / 60) ** 2))                      # chlorosis

edge = 1 / (1 + np.exp(-(wl - (715 - 12 * s)) / 12))

nir = (0.48 * (1 - 0.3 * s)) * edge

water = 1 - (0.06 * np.exp(-((wl - 970) / 25) ** 2) + 0.1 * np.exp(-((wl - 1200) / 40) ** 2)

+ 0.6 * np.exp(-((wl - 1450) / 60) ** 2) + 0.7 * np.exp(-((wl - 1940) / 70) ** 2)) * (1 - 0.3 * s)

swir = np.where(wl > 1300, (0.35 - 0.12 * (wl - 1300) / 1200) + 0.08 * s, 1)

return (vis * (1 - edge) + nir * np.where(wl > 1300, swir / 0.48, 1)) * water

return soil, veg(0.0), veg(1.0), np.full_like(wl, 0.55)

def block_mean(a, f):

h, w = a.shape[0] // f * f, a.shape[1] // f * f

return a[:h, :w].reshape(h // f, f, w // f, f).mean((1, 3))

def main():

for d in [C.ROOT]:

d.mkdir(parents=True, exist_ok=True)

layout = build_layout_table()

layout.to_csv(C.LAYOUT_CSV, index=False)

plots = build_plot_polygons(layout)

plots.set_geometry("geometry").drop(columns="geometry_inner").to_file(C.PLOTS_FILE, layer="plots")

sus = {"V1": 1.4, "V2": 1.0, "V3": 0.7}

age = {"D1": 1.2, "D2": 1.0, "D3": 0.8, "D4": 0.6}

tf = [0.25, 0.55, 0.85, 1.15]

gt = []

for _, p in layout.iterrows():

base = rng.lognormal(0, 0.35)

for t, date in zip(tf, C.DATES):

di = 0.0 if not p.inoculated else float(np.clip(32 * sus[p.variety] * age[p.inoc_age] * t * base, 0.5, 95))

gt.append(dict(plot_id=p.plot_id, date=date, DI=round(di, 1)))

gt = pd.DataFrame(gt)

gt.to_csv(C.GROUND_TRUTH_CSV, index=False)

pd.DataFrame(dict(date=C.DATES, air_temp_C=[AIR[d] for d in C.DATES])).to_csv(C.WEATHER_CSV, index=False)

np.savetxt(C.HS_WAVELENGTH_FILE, WL, fmt="%.1f")

tr = from_origin(X0, Y0, FINE, FINE)

pid = rasterize([(g, int(i)) for g, i in zip(plots.geometry, plots.plot_id)], (H, W), transform=tr, dtype="int32")

soil, healthy, diseased, white = spectra()

for date in C.DATES:

di_map = dict(zip(gt[gt.date == date].plot_id, gt[gt.date == date].DI))

s_fine = np.vectorize(lambda i: min(di_map.get(i, 0) / 50, 1.0))(pid) * (pid > 0)

noise = gaussian_filter(rng.standard_normal((H, W)), 6)

veg = (pid > 0) & (noise > np.quantile(noise, 0.2) + 0.25 * s_fine * noise.std())

pust = veg & (rng.random((H, W)) < 0.25 * s_fine)

for sensor, f in FACTORS.items():

fv, fp, s = block_mean(veg.astype(float), f), block_mean(pust.astype(float), f), block_mean(s_fine, f)

h, w = fv.shape

prof = dict(driver="GTiff", height=h, width=w, crs=C.CRS, transform=from_origin(X0, Y0, FINE * f, FINE * f))

spec = ((fv - fp)[None] * healthy[:, None, None] + ((fv - fp) * s)[None] * (diseased - healthy)[:, None, None]

+ fp[None] * white[:, None, None] + (1 - fv)[None] * soil[:, None, None])

if sensor == "HS":

cube = spec + rng.normal(0, 0.004, spec.shape)

write_raster(C.raw_path("HS", date), np.clip(cube, 0, 1) * 10000, prof, dtype="uint16", nodata=0)

elif sensor == "MS":

idx = [int(np.argmin(np.abs(WL - c))) for c in C.MS_BANDS.values()]

write_raster(C.raw_path("MS", date), spec[idx] + rng.normal(0, 0.005, (5, h, w)), prof)

elif sensor == "RGB":

idx = [int(np.argmin(np.abs(WL - c))) for c in (640, 550, 470)]

rgb = np.clip(spec[idx] / 0.6 * 255 + rng.normal(0, 4, (3, h, w)), 0, 255)

write_raster(C.raw_path("RGB", date), rgb, prof, dtype="uint8", nodata=None)

else:

at = AIR[date]

t_c = fv * (at - 3 + 4 * s) + (1 - fv) * (at + 9) + rng.normal(0, 0.3, (h, w))

shifted = prof.copy()

shifted["transform"] = from_origin(X0 + 0.24, Y0, FINE * f, FINE * f)   # 24 cm registration error

write_raster(C.raw_path("TIR", date), t_c + 273.15, shifted)

print(f"synthetic {date}: written")

if __name__ == "__main__":

main()

## 12.3 Test run

The whole pipeline ran without errors on the synthetic data (Steps 1–6 with the full settings; Step 7–8 with RW_FAST=1: one repeat, fewer trees, SFFS up to 6 features). The numbers below only confirm that the code produces every output. They say nothing about real accuracy: the synthetic spectra are simplified and much easier to separate than real white rust.

| Step | What was checked on the synthetic data | Result |
| --- | --- | --- |
| 1 Layout | 48 plot polygons built; layout check run | Works; R1 duplicate/missing treatments reported |
| 2 Pre-processing | Unit conversion (HS ÷10,000, RGB ÷255, TIR K→°C), bad-band removal, smoothing, co-registration | Works; 24 cm thermal offset recovered (6 MS pixels) on all 4 dates |
| 3 Masking | NDVI/ExG Otsu masks, shadow removal, thermal mask from MS | Works |
| 4 Unmixing | VCA endmembers named soil / healthy / diseased / shadow; FCLS abundances | Works; 21,100 pixels unmixed per date |
| 5 Features | 192 rows per sensor: MS 19, HS 150, RGB 30, TIR 17 features | Works; low-cover fallback used for the most severe plots |
| 6 Phase 1 | QC, KW + FDR, JM, ReliefF, RF importance, Boruta, consensus, stability, clustering | Works; DSFs: MS 8, HS 66, RGB 6, TIR 7 |
| 7 Nested CV | Per-fold Phase 1, VIF, SFFS, tuning; single sensors and MS+TIR fusion; SVM and KNN | Works |
| 8 Outputs | Summary, fusion gain + McNemar, class report, robust features, final model, SHAP (RF, SVM, KNN), maps, DI regression | Works |

- The full run of Step 7 (8 configurations × 3 classifiers × 4 folds × 3 repeats) takes several hours on a laptop; it was tested here in reduced form (2 outer folds, 1 repeat).

- The joblib library may print “leaked folder objects” warnings at the end of Step 7; they are harmless.

- Accuracies on synthetic data are not reported because they are meaningless for real white rust.

# 13. Using the code with your real data: checklist and tips

- Put the 16 orthomosaics in project/01_orthomosaics/<sensor>/<sensor>_<date>.tif, with the dates written as in config.DATES.

- Provide ground_truth.csv, weather.csv and, if needed, hs_wavelengths.txt and calibration_panels.csv.

- Run Step 1; check the layout warning and overlay plots.gpkg on an orthomosaic in QGIS.

- Run Step 2; check coregistration_log.csv (shifts should be small) and open one preprocessed raster of each sensor.

- Run Step 3; inspect masks for a healthy and a severe plot; set NDVI_MIN if severe canopy is removed.

- Run Step 4; check the endmember plot; adjust N_ENDMEMBERS if two endmembers are duplicates.

- Run Step 5; open the four feature tables; check the low-cover messages and that values are in plausible ranges (NDVI 0.2–0.9, canopy temperature within a few degrees of air temperature).

- Run Step 6; read the feature tables and the JM scalogram; these are the Objective 1 results.

- Run Step 7 with RW_FAST=1, then without it for the thesis (this can take several hours on a laptop; it uses all CPU cores).

- Report the nested-CV performance (mean ± SD over repeats), fusion gain with McNemar p-values, the final OFCs, robust features, SHAP sensor contributions and the maps.

## 13.1 Practical notes

- Memory. Step 2 reads a whole raster at once. For very large hyperspectral cubes, clip the orthomosaic to the field boundary first (e.g., gdalwarp -cutline field.gpkg -crop_to_cutline).

- Few observations per class. With 48 plots and 4 dates, some severity classes may have very few plot-dates. Report per-class results and also the binary task (healthy vs diseased: use diseased as the target), or merge adjacent classes if a class has fewer than about 10 observations.

- Time. The modelling cost is dominated by SFFS with Random Forest. Reduce SFFS_MAX_FEATURES or the number of trees during development; use the full settings for the final run.

- Reproducibility. All random steps use RANDOM_STATE. Report package versions (pip freeze > requirements.txt).

- Same steps for new fields. For Tier 1 (IIRMR Bharatpur) and Tier 2 (farmers’ fields) run Steps 2–5 with the same settings, then predict with the saved final model; never re-select features on the validation fields.

# References

Araújo, M. C. U., Saldanha, T. C. B., Galvão, R. K. H., Yoneyama, T., Chame, H. C., & Visani, V. (2001). The successive projections algorithm for variable selection in spectroscopic multicomponent analysis. Chemometrics and Intelligent Laboratory Systems, 57(2), 65–73.

Benjamini, Y., & Hochberg, Y. (1995). Controlling the false discovery rate. Journal of the Royal Statistical Society: Series B, 57(1), 289–300.

Guizar-Sicairos, M., Thurman, S. T., & Fienup, J. R. (2008). Efficient subpixel image registration algorithms. Optics Letters, 33(2), 156–158.

Haralick, R. M., Shanmugam, K., & Dinstein, I. (1973). Textural features for image classification. IEEE Transactions on Systems, Man, and Cybernetics, SMC-3(6), 610–621.

Heinz, D. C., & Chang, C.-I. (2001). Fully constrained least squares linear spectral mixture analysis method for material quantification in hyperspectral imagery. IEEE Transactions on Geoscience and Remote Sensing, 39(3), 529–545.

Kononenko, I. (1994). Estimating attributes: Analysis and extensions of RELIEF. European Conference on Machine Learning, 171–182.

Kursa, M. B., & Rudnicki, W. R. (2010). Feature selection with the Boruta package. Journal of Statistical Software, 36(11), 1–13.

Lundberg, S. M., & Lee, S.-I. (2017). A unified approach to interpreting model predictions. Advances in Neural Information Processing Systems, 30.

Meinshausen, N., & Bühlmann, P. (2010). Stability selection. Journal of the Royal Statistical Society: Series B, 72(4), 417–473.

Nascimento, J. M. P., & Bioucas-Dias, J. M. (2005). Vertex component analysis: A fast algorithm to unmix hyperspectral data. IEEE Transactions on Geoscience and Remote Sensing, 43(4), 898–910.

Otsu, N. (1979). A threshold selection method from gray-level histograms. IEEE Transactions on Systems, Man, and Cybernetics, 9(1), 62–66.

Pedregosa, F., et al. (2011). Scikit-learn: Machine learning in Python. Journal of Machine Learning Research, 12, 2825–2830.

Pontes, M. J. C., et al. (2005). The successive projections algorithm for spectral variable selection in classification problems. Chemometrics and Intelligent Laboratory Systems, 78(1–2), 11–18.

Pudil, P., Novovičová, J., & Kittler, J. (1994). Floating search methods in feature selection. Pattern Recognition Letters, 15(11), 1119–1125.

Raschka, S. (2018). MLxtend: Providing machine learning and data science utilities and extensions to Python’s scientific computing stack. Journal of Open Source Software, 3(24), 638.

Roberts, D. R., et al. (2017). Cross-validation strategies for data with temporal, spatial, hierarchical, or phylogenetic structure. Ecography, 40(8), 913–929.

Savitzky, A., & Golay, M. J. E. (1964). Smoothing and differentiation of data by simplified least squares procedures. Analytical Chemistry, 36(8), 1627–1639.

Varma, S., & Simon, R. (2006). Bias in error estimation when using cross-validation for model selection. BMC Bioinformatics, 7, 91.

Woebbecke, D. M., Meyer, G. E., Von Bargen, K., & Mortensen, D. A. (1995). Color indices for weed identification under various soil, residue, and lighting conditions. Transactions of the ASAE, 38(1), 259–269.

See the methodology document for the references of the individual vegetation indices, statistical tests and classifiers. Verify volumes and pages before citing.